OC-1 MANUAL TESTING

  - TESTCASE- LLDP

## Topology

#### Configure and verify LLDP global and interface CLI

| Test ID | 4.16.1 |
| --- | --- |
| Feature | LLDP |
| Test Case | Configure and verify LLDP global and interface CLI |
| Test Item | CLI Configuration |
| Test Objective | Validate LLDP global, interface, TLVs, timers, management address configuration |
| Test Setup | DUT(s) running SONiC+; required peer devices / traffic generator as applicable; management access (SSH/console); logging enabled. |
| Pre-conditions | 1) Device reachable via management. 2) Baseline configuration saved. 3) Interfaces are cabled as per topology. 4) Counters/logs cleared before execution (if applicable). |
| Test Procedure | 1. Configure prerequisite settings for LLDP. 2. Execute the action described in the one-liner. 3. Verify using relevant show/debug commands. 4. Roll back configuration and re-verify baseline. |
| Expected Result | LLDP neighbors discovered with correct TLVs; stats updated |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : PASS |

Logs: Configuration and Verification

sonic# show lldp neighbor Display LLDP neighbor information statistics Display LLDP statistics information table Display LLDP table information sonic(config)# lldp enable sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# lldp enable <cr> sonic(conf-if-Ethernet8)# lldp enable sonic(conf-if-Ethernet8)# exit sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP --more— sonic# show lldp statistics LLDP Statistics --------------------------------- sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:89:10 sonic fortyGigE0/20 Ethernet20 52:54:00:c3:89:10 sonic fortyGigE0/24 Ethernet24

#### Verify LLDP neighbor discovery

| Test ID | 4.16.2 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP neighbor discovery |
| Test Item | Function |
| Test Objective | Enable LLDP and verify neighbor discovery on peer connection |
| Test Procedure | 1. Configure prerequisite settings for LLDP. 2. Execute the action described in the one-liner. 3. Verify using relevant show/debug commands. 4. Roll back configuration and re-verify baseline. |
| Expected Result | Neighbor discovered with correct TLVs |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : PASS |

Logs: Configuration and Verification

admin@sonic:~$ admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# lldp enable sonic(conf-if-Ethernet8)# lldp TRANSMIT sonic(conf-if-Ethernet8)# lldp RECEIVE sonic(conf-if-Ethernet8)# exit sonic# # Repeat the same steps on DUT-B Verifying LLDP status sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Verify LLDP neighbor status per interface : sonic# show lldp neighbor Ethernet 8 Traceback (most recent call last): File "/usr/sbin/cli/sonic-cli-lldp.py", line 661, in <module> status = run(func, sys.argv[2:]) ^^^^^^^^^^^^^^^^^^^^^^^ File "/usr/sbin/cli/sonic-cli-lldp.py", line 617, in run response = invoke_api(fn, args) ^^^^^^^^^^^^^^^^^^^^ File "/usr/sbin/cli/sonic-cli-lldp.py", line 274, in invoke_api if "openconfig-lldp:interface" in response.content: ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ TypeError: argument of type 'NoneType' is not iterable sonic# #Getting error while displaying ethernet 8 LLDP status

#### Verify transmit-only, receive-only and both modes

| Test ID | 4.16.3 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify transmit-only, receive-only and both modes |
| Test Item | Functional |
| Test Objective | Validate LLDP transmit/receive behavior in different modes |
| Expected Result | Neighbors appear/disappear as per mode |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : FAIL |

Logs: Configuration and Verification

On VS2: Configure only LLDP RECEIVE sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# no lldp RECEIVE sonic(conf-if-Ethernet8)# exit Verifying LLDP neighbor info sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 Expected when no lldp receive is configured: - DUT should NOT learn any neighbors on Ethernet8 - Output of show lldp neighbor should NOT show entries for Ethernet8 Actual Behavior: - DUT still shows LLDP neighbor entries on Ethernet8 - LLDP receive filtering is NOT working This means LLDP RECEIVE mode is not being honored. On VS2: Configure only LLDP RECEIVE and verify on peer DUT admin@sonic:~$ admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# lldp RECEIVE sonic(conf-if-Ethernet8)# no lldp TRANSMIT sonic(conf-if-Ethernet8)# exit sonic# Verifying LLDP neighbor info on Peer DUT sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 Expected when no lldp transmit is configured: - On DUT1: You should see the peer as a neighbor (RX still enabled). - On Peer DUT: You should NOT see DUT1 as a neighbor on the link to Ethernet8 (because DUT1 is not transmitting). OBSERVED BEHAVIOR: - Peer DUT still sees DUT1 as an LLDP neighbor. This means LLDP TRANSMIT mode is not being honored.

#### Verify per-interface LLDP enable/disable

| Test ID | 4.16.4 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify per-interface LLDP enable/disable |
| Test Item | Functional |
| Test Objective | Validate LLDP neighbor discovery with per-interface control |
| Expected Result | Neighbor shown only on enabled interfaces |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : FAIL |

Logs: Configuration and Verification

Enable LLDP on Ethernet12 on DUT A and Ethernet16 on DUT B On DUT-A sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# lldp <RECEIVE/TRANSMIT> Configure mode enable Enable LLDP at interface level med-tlv-select Enables LLDP-MED TLV advertisement tlv-select Enables LLDP TLV advertisement tlv-set Configures IPv4/IPv6 management address vlan-name-tlv Configures vlan-list for vlan-name TLV advertisement sonic(conf-if-Ethernet12)# lldp enable sonic(conf-if-Ethernet12)# lldp enable med-tlv-select RECEIVE tlv-select tlv-set TRANSMIT vlan-name-tlv sonic(conf-if-Ethernet12)# lldp TRANSMIT sonic(conf-if-Ethernet12)# lldp RECEIVE sonic(conf-if-Ethernet12)# On DUT-B admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 16 sonic(conf-if-Ethernet16)# lldp enable sonic(conf-if-Ethernet16)# lldp TRANSMIT sonic(conf-if-Ethernet16)# lldp RECEIVE sonic(conf-if-Ethernet16)# exit sonic# Baseline Verification: On DUT-A sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- On DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Now Disable LLDP on Ethernet8 on DUT-A sonic(conf-if-Ethernet8)# no lldp enable sonic(conf-if-Ethernet8)# Verifying behaviour: On DUT-A sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Expected: - Ethernet12 should STILL show a neighbor - Ethernet8 should NO LONGER show any neighbor Observed: - Ethernet8 still showing neighbor. On DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Expected: - DUTB should stop seeing DUT1 on Ethernet8 - But should still see DUT1 on the port connected to Ethernet12 Observed: - Getting LLDp info from Ethernet8 as well as Ethernet12.

#### Verify LLDP timers and multiplier

| Test ID | 4.16.5 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP timers and multiplier |
| Test Item | Functional |
| Test Objective | Validate LLDP timer and multiplier configuration |
| Expected Result | Neighbor shown only on enabled interfaces |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : FAIL : CLI working but LLDP frames received do NOT reflect the configured timer + multiplier. |

Logs: Configuration and Verification

Configure LLDP Timers on DUTA-A sonic(config)# lldp timer 20 sonic(config)# lldp multiplier <1..10> Multiplier value (1 to 10) sonic(config)# lldp multiplier 4 sonic(config)#exit Verify on DUT-B: admin@sonic:~$ sudo tcpdump -i Ethernet8 -vv -s 1500 ether proto 0x88cc tcpdump: listening on Ethernet8, link-type EN10MB (Ethernet), snapshot length 1500 bytes 10:07:13.874911 LLDP, length 260 Chassis ID TLV (1), length 7 Subtype MAC address (4): 52:54:00:c3:47:10 (oui Unknown) 0x0000: 0452 5400 c347 10 Port ID TLV (2), length 13 Subtype Local (7): fortyGigE0/8 0x0000: 0766 6f72 7479 4769 6745 302f 38 Time to Live TLV (3), length 2: TTL 120s 0x0000: 0078 System Name TLV (5), length 5: sonic 0x0000: 736f 6e69 63 System Description TLV (6), length 142 SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683 Restoring Config: admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# lldp timer 30 sonic(config)# lldp multiplier 4 sonic(config)#

#### Verify system-name/description and management TLV

| Test ID | 4.16.6 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify system-name/description and management TLV |
| Test Item | Functional |
| Test Objective | Validate system-name, description and management-address TLVs |
| Expected Result | Neighbor sees SysName, SysDescr, Mgmt IP |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution): FAIL: CLI working but Neighbor sees SysName, SysDescr but management address TLV missing. |

Logs: Configuration and Verification

STEP 1 — Configure System Name and System Description on DUT-A sonic(config)# lldp system-name supermicro sonic(config)# lldp system-description "DUT-A LLDP Test Device" sonic(config)# exit sonic# STEP 2 — Configure Management Address TLV (Mgmt IP) on DUT-A sonic(config)# lldp tlv-select system-capabilities sonic(config)# lldp tlv-select management-address sonic(config)# interface Ethernet 0 sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv4 192.168.100.64 sonic(conf-if-Ethernet0)# STEP 3 — Verify TLVs on DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 -----------------------------------------------------------

#### Verify VLAN specific LLDP TLVs

| Test ID | 4.16.7 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify VLAN specific LLDP TLVs |
| Test Item | Functional |
| Test Objective | Validate VLAN Name TLV advertisement and limits |
| Expected Result | Only allowed VLAN TLVs advertised |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL: CLI working but Unable to configure Vlan. VLAN TLVs missing in LLDP neighbor info on Peer DUT |

Logs: Configuration and Verification

STEP 1 — Configure VLAN(s) on DUT-A: admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# vlan 100 ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface vlan sonic(config)# vlan100 ^ % Error: Invalid input detected at "^" marker. sonic(config)# # Could not configure VLAN in ISCLI STEP 2 — Enable LLDP VLAN Name TLV: sonic(conf-if-Ethernet8)# lldp tl tlv-select tlv-set sonic(conf-if-Ethernet8)# lldp tlv-select link-aggregation 802.1 Link Aggregation TLV max-frame-size 802.3 Maximum Frame size TLV port-vlan-id 802.1 PORT VLAN ID TLV power-management 802.3 TLV vlan-name 802.1 VLAN NAME TLV sonic(conf-if-Ethernet8)# lldp tlv-select po port-vlan-id power-management sonic(conf-if-Ethernet8)# lldp tlv-select port-vlan-id sonic(conf-if-Ethernet8)# lldp vlan-name-tlv allowed Vlan 100,200 sonic(conf-if-Ethernet8)#exit Verify LLDP VLAN TLVs on DUT-B: sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 -----------------------------------------------------------

#### Rapid enable/disable LLDP

| Test ID | 4.16.8 |
| --- | --- |
| Feature | LLDP |
| Test Case | Rapid enable/disable LLDP |
| Test Item | Negative |
| Test Objective | Validate LLDP stability under rapid enable/disable |
| Expected Result | No stale LLDP entries |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) |

Logs: Configuration and Verification

#### Verify LLDP after save and reboot

| Test ID | 4.16.10 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP after save and reboot |
| Test Item | Persistence |
| Test Objective | Validate LLDP persistence after reboot |
| Expected Result | LLDP works post reboot |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution): PASS : Mgmt ip configuration retained after save and reboot (but in Sudo Mode).Mgmt ip field missing in ISCLI |

Logs: Configuration and Verification

On DUT-A Configure Management Address TLV (Mgmt IP) on DUT-A sonic(config)# lldp tlv-select system-capabilities sonic(config)# lldp tlv-select management-address sonic(config)# interface Ethernet 0 sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv4 192.168.100.64 sonic(conf-if-Ethernet0)# admin@sonic:~$ sudo config save -y Running command: /usr/local/bin/sonic-cfggen -d --print-data > /etc/sonic/config_db.json admin@sonic:~$ sudo reboot requested COLD shutdown On DUT-B immediately(After save and reboot on DUT-A) No DUT-A LLDP info received on DUT B. No Stale LLDP info Only LLDP Info of another connected DUT with DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- sonic# admin@sonic:~$ sonic-cli sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# admin@sonic:~$ sudo lldpctl show neighbors ports Ethernet8 ------------------------------------------------------------------------------- LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet8, via: LLDP, RID: 3, Time: 0 day, 00:08:55 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/8 PortDescr: Ethernet8 TTL: 120 PMD autoneg: supported: no, enabled: no MAU oper type: 10BaseTFD - UTP MAU, full duplex mode ------------------------------------------------------------------------------- admin@sonic:~$

#### Verify LLDP neighbor detail TLVs (per interface)

| Test ID | 4.16.11 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP neighbor detail TLVs (per interface) |
| Test Item | Functional |
| Test Objective | Validate full neighbor TLVs (Chassis ID, Port ID, TTL, SysName, SysDescr, SysCaps, Mgmt-Addr, Port-Descr). |
| Expected Result | All mandatory TLVs are decoded correctly and match the peer; no crashes or unknown standard TLVs. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : FAIL : Although some TLVs are present, the IS-CLI output is missing mandatory fields for this test (TTL value, System Capabilities, Management Address) and does not identify the local port clearly. Local port identification missing. |

Logs: Configuration and Verification

Config on DUT-A admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# lldp sys system-description system-name sonic(config)# lldp system-name supermicro sonic(config)# interface Ethernet 0 sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv4 192.168.100.229 sonic(conf-if-Ethernet0)# exit sonic(config)# lldp system-description "OCLI TESTING " sonic(conf-if-Ethernet8)# lldp tlv-select max-frame-size Verify on DUT-B before save and reboot of DUT-A sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- sonic# sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# show lldp table Verify TLVs and LLDP neighbor details after DUT_A reboot sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 sonic# admin@sonic:~$ sonic-cli sonic# show lldp neighbor Ethernet 8 Traceback (most recent call last): File "/usr/sbin/cli/sonic-cli-lldp.py", line 661, in <module> status = run(func, sys.argv[2:]) ^^^^^^^^^^^^^^^^^^^^^^^ File "/usr/sbin/cli/sonic-cli-lldp.py", line 617, in run response = invoke_api(fn, args) ^^^^^^^^^^^^^^^^^^^^ File "/usr/sbin/cli/sonic-cli-lldp.py", line 274, in invoke_api if "openconfig-lldp:interface" in response.content: ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ TypeError: argument of type 'NoneType' is not iterable sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- TLV Required; Present?; Where Seen. Chassis ID; ✔; ChassisID: MAC. Port ID; ✔; PortID: fortyGigE0/8. TTL; ❌; Value missing. SysName; ✔; SysName: sonic. SysDescr; ✔; SONiC version string. SysCaps; ❌; Missing. Mgmt-IP; ❌; Missing. Port Description; ✔; PortDescr: Ethernet8.

#### Verify Port Description TLV publication

| Test ID | 4.16.12 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify Port Description TLV publication |
| Test Item | Functional |
| Test Objective | Ensure port alias/description is advertised and received by neighbor. |
| Expected Result | Peer sees the configured Port Description; updates occur within TTL without stale values. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL — Peer sees the default Port Description, not the configured value; updates don’t take effect per IS-CLI neighbor output. |

Logs: Configuration and Verification

On DUT-A admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# lldp enable sonic(config)# lldp tlv-select <management-address/system-capabilities> TLV to be advertised sonic(config)# lldp tlv-select system-capabilities <cr> sonic(config)# lldp tlv-select system-capabilities sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# lldp TRANSMIT sonic(conf-if-Ethernet8)# lldp RECEIVE sonic(conf-if-Ethernet8)# description LINK_TO_PEER sonic(conf-if-Ethernet8)# exit Verify on DUT-B after save and Reboot on DUT-A: sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 -----------------------------------------------------------

#### Verify System Capabilities TLV

| Test ID | 4.16.13 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify System Capabilities TLV |
| Test Item | Functional |
| Test Objective | Ensure advertised System Capabilities/Enabled Capabilities reflect actual device role. |
| Expected Result | Capabilities and Enabled Capabilities match expectations for the platform/SKU. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution): FAIL — System Capabilities TLV not visible in IS-CLI LLDP neighbor output. |

Logs: Configuration and Verification

On DUT-A sonic(config)# exit sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ON DUT-B Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 -----------------------------------------------------------

#### Verify Management Address selection order

| Test ID | 4.16.14 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify Management Address selection order |
| Test Item | Functional |
| Test Objective | Confirm the correct management address is advertised when multiple addresses exist. |
| Expected Result | Advertised Management Address follows selection policy and remains stable. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL: Mgmt address not adverstised . Also forcefully configured mgmt ip address has no effect on peer DUT |

Logs: Configuration and Verification

Priority for LLDP Mgmt Address TLV: - Admin-configured management address (highest priority) - Mgmt0 IPv4 address (if present) - Loopback IPv4 - IPv6 management address - Any other IPv6 STEP 1 — Ensure baseline LLDP works (both DUTs) DUT-A sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:89:10 sonic fortyGigE0/20 Ethernet20 52:54:00:c3:89:10 sonic fortyGigE0/24 Ethernet24 sonic# DUT-B sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# STEP 2 — Add multiple management addresses on DUT‑A Ethernet124 10.0.0.62/31 up/up ARISTA16T0 10.0.0.63 Loopback0 10.1.0.1/32 up/up N/A N/A docker0 240.127.1.1/24 up/down N/A N/A eth0 192.168.100.64/24 up/up N/A N/A lo 127.0.0.1/16 up/up N/A N/A admin@sonic:~$ sonic-cli sonic# 1) Mgmt0 IPv4 2) Loopback0 ipv4 admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 0 sonic(conf-if-Ethernet0)# ip address 192.168.100.229/24 sonic(conf-if-Ethernet0)# exit sonic# configure terminal sonic(config)# interface loopback0 ^ % Error: Invalid input detected at "^" marker. sonic(config)# STEP 3 — Configure LLDP Management Address TLV (IS-CLI) Optional (force explicitly configured mgmt address): sonic(config)# interface Ethernet 0 sonic(conf-if-Ethernet0)# lldp tl tlv-select tlv-set sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv4 IPv4 management address ipv6 IPv6 management address sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv ipv4 ipv6 sonic(conf-if-Ethernet0)# lldp tlv-set management-address ipv4 192.168.100.229 sonic(conf-if-Ethernet0)# STEP 4 — Verify Management Address Advertised On DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- # No Mgmt ip field present in this command output admin@sonic:~$ sudo lldpcli show neighbors ports Ethernet8 ------------------------------------------------------------------------------- LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet8, via: LLDP, RID: 6, Time: 0 day, 03:49:27 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/8 PortDescr: Ethernet8 TTL: 120 ------------------------------------------------------------------------------- admin@sonic:~$ # Mgmt ip found same, no change observed on DUT-B even after restarting lldp container(sudo docker restart lldp)

#### Verify TTL/hold behavior at default timers

| Test ID | 4.16.15 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify TTL/hold behavior at default timers |
| Test Item | Functional |
| Test Objective | Validate neighbor age-out aligns with tx-interval × multiplier. |
| Expected Result | Observed timeout closely matches configured TTL; no excessive delay or premature age-out. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL: TTL Value not shown in LLDP neighbor output. Also no impact of disabling LLDP globally or per interface on peer DUT. |

Logs: Configuration and Verification

STEP 1 — Disable LLDP Transmission on DUT-A (to stop TX) admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# lldp enable sonic(conf-if-Ethernet12)# lldp RECEIVE sonic(conf-if-Ethernet12)# no lldp TRANSMIT sonic(conf-if-Ethernet12)# admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# lldp TRANSMIT sonic(conf-if-Ethernet12)# exit sonic# configure terminal Disabling LLDP globally: sonic(config)# no lldp enable sonic(config)# Verify on Peer DUT,LLDP Frames must stop sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP --more— # admin@sonic:~$ sudo tcpdump -i Ethernet12 -vv -s 1500 ether proto 0x88cc Command displayed continuous LLDp frames on DUT B until stopped manually.

#### Verify LLDP on LAG member interfaces

| Test ID | 4.16.16 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP on LAG member interfaces |
| Test Item | Functional |
| Test Objective | Validate discovery behavior on physical members of a Link Aggregation (LAG). |
| Expected Result | Neighbors discovered per member port; no LLDP entry on the logical LAG interface. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) :Not Applicable |

Logs: Configuration and Verification

ON DUT-A admin@sonic:~$ admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# lldp enable sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# lldp TRANSMIT sonic(conf-if-Ethernet8)# lldp RECEIVE sonic(conf-if-Ethernet8)# exit sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# lldp TRANSMIT sonic(conf-if-Ethernet12)# lldp RECEIVE sonic(conf-if-Ethernet12)# exit sonic# Configure LAG (PortChannel) sonic(config)# interface portchannel10 ^ % Error: Invalid input detected at "^" marker. sonic(config)#

#### Verify LLDP continuity during MTU change

| Test ID | 4.16.17 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP continuity during MTU change |
| Test Item | Functional |
| Test Objective | Ensure LLDP operation persists across MTU changes on the port. |
| Expected Result | No LLDP disruption due solely to MTU change; neighbor table remains consistent. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) PASS: No impact on LLDP operation found on DUT-B post MTU change on DUT-A |

Logs: Configuration and Verification

ON Peer DUT STEP 1-Verify Baseline LLDP Neighbor (Before MTU Change) sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP STEP 2-Change MTU on DUT-A (Test Action) sonic# configure terminal sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# mtu <1312..9216> MTU of the interface sonic(conf-if-Ethernet8)# mtu 1500 sonic(conf-if-Ethernet8)# exit sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# mtu 1500 sonic(conf-if-Ethernet12)# exit sonic# Verify on Peer DUT LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet8, via: LLDP, RID: 6, Time: 0 day, 05:57:41 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/8 PortDescr: Ethernet8 TTL: 120 ------------------------------------------------------------------------------- LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet16, via: LLDP, RID: 6, Time: 0 day, 05:57:31 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/12 PortDescr: Ethernet12 TTL: 120 ------------------------------------------------------------------------------- admin@sonic:~$ sonic-cli sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# sonic# exit admin@sonic:~$ sudo reboot requested COLD shutdown /var/log: 201.9 MiB (211714048 bytes) trimmed on /dev/loop1

#### Verify per-port TLV enable/disable

| Test ID | 4.16.18 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify per-port TLV enable/disable |
| Test Item | Functional |
| Test Objective | Validate that specific TLVs (e.g., Management-Address) can be toggled per port. |
| Expected Result | TLV visibility matches configured state; no residual or ghost TLVs. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL- Functionality not working in ISCLI.No impact on peer DUT seen even after disabling lldp tlv-select management-address on DUT-A in sudo mode |

Logs: Configuration and Verification

ON DUT-A sonic(config)# lldp <RECEIVE/TRANSMIT> Configure mode enable Enable LLDP at global level multiplier Configure LLDP multiplier value (1 to 10) system-description Configure LLDP system description system-name Configure LLDP system name timer Configure LLDP hello time value in seconds (5 to 254 sec) tlv-select Enable LLDP TLVs to be advertised sonic(config)# lldp tlv-select <management-address/system-capabilities> TLV to be advertised # Can toggle only Mgmt-address and System-Capabilities TLV admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# lldp tlv-select management-address sonic(config)# no lldp tlv-select management-address sonic(config)# exit sonic# exit Restart LLDP: admin@sonic:~$ sudo docker restart lldp lldp admin@sonic:~$ ON Peer DUT-B admin@sonic:~$ sudo lldpcli show neighbors ports Ethernet8 ------------------------------------------------------------------------------- LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet8, via: LLDP, RID: 7, Time: 0 day, 00:31:13 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/8 PortDescr: Ethernet8 TTL: 120 ------------------------------------------------------------------------------- admin@sonic:~$ sudo lldpcli show neighbors ports Ethernet16 ------------------------------------------------------------------------------- LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet16, via: LLDP, RID: 7, Time: 0 day, 00:31:51 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/12 PortDescr: Ethernet12 TTL: 120 ------------------------------------------------------------------------------- admin@sonic:~$

#### Verify LLDP statistics counters

| Test ID | 4.16.19 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP statistics counters |
| Test Item | Functional |
| Test Objective | Confirm Tx/Rx/AgeOut/Discard counters increment predictably. |
| Expected Result | Counters reflect operations; no abnormal error spikes or counter wrap. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL-show lldp statistics producing no output in ISCLi |

Logs: Configuration and Verification

On DUT-B # show lldp statistics command giving no output admin@sonic:~$ sonic-cli sonic# show lldp statistics LLDP Statistics --------------------------------- sonic# In Sudo Mode: admin@sonic:~$ sudo lldpcli show statistics ports Ethernet16 ------------------------------------------------------------------------------- LLDP statistics: ------------------------------------------------------------------------------- Interface: Ethernet16 Transmitted: 3940 Received: 3937 Discarded: 0 Unrecognized: 0 Ageout: 5 Inserted: 16 Deleted: 15 ------------------------------------------------------------------------------- admin@sonic:~$ sudo lldpcli show statistics ports Ethernet8 ------------------------------------------------------------------------------- LLDP statistics: ------------------------------------------------------------------------------- Interface: Ethernet8 Transmitted: 3940 Received: 3951 Discarded: 0 Unrecognized: 0 Ageout: 5 Inserted: 16 Deleted: 15 ------------------------------------------------------------------------------- admin@sonic:~$ sudo lldpcli show statistics ports Ethernet8 ------------------------------------------------------------------------------- LLDP statistics: ------------------------------------------------------------------------------- Interface: Ethernet8 Transmitted: 3957 Received: 3968 Discarded: 0 Unrecognized: 0 Ageout: 5 Inserted: 16 Deleted: 15 ------------------------------------------------------------------------------- admin@sonic:~$ sudo lldpcli show statistics ports Ethernet16 ------------------------------------------------------------------------------- LLDP statistics: ------------------------------------------------------------------------------- Interface: Ethernet16 Transmitted: 3957 Received: 3955 Discarded: 0 Unrecognized: 0 Ageout: 5 Inserted: 16 Deleted: 15 ------------------------------------------------------------------------------- admin@sonic:~$

#### Verify SNMP LLDP MIB parity (optional)

| Test ID | 4.16.20 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify SNMP LLDP MIB parity (optional) |
| Test Item | Functional |
| Test Objective | Validate LLDP data parity across CLI and SNMP MIB tables (if SNMP is enabled). |
| Expected Result | SNMP values match CLI content for neighbor entries and TLVs. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) NOT FEASIBLE/APPLICABLE |

#### Verify IPv6 Management Address TLV

| Test ID | 4.16.21 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify IPv6 Management Address TLV |
| Test Item | Functional |
| Test Objective | Validate advertisement and decoding of IPv6 management address. |
| Expected Result | Peer displays the correct IPv6 Mgmt Address TLV for the DUT. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL: NO IPV6 Mgmt field visible in show lldp neighbor output on Peer DUT |

Logs: Configuration and Verification

ON DUT-A sonic(conf-if-Ethernet0)# ipv6 address fd10:20::25/128 sonic(conf-if-Ethernet0)# exit sonic# configure terminal sonic(config)# lldp tlv-select management-address sonic(config)# exit sonic# exit Verify IPv6 Mgmt TLV on DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP --more--

#### Verify LLDP over different media

| Test ID | 4.16.22 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify LLDP over different media |
| Test Item | Functional |
| Test Objective | Ensure LLDP operation across fiber and copper (DAC) connections. |
| Expected Result | Discovery and TLVs operate identically across media types. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) Not Applicable |

#### Verify neighbor flush and re-discovery on admin down/up

| Test ID | 4.16.23 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify neighbor flush and re-discovery on admin down/up |
| Test Item | Functional |
| Test Objective | Ensure no stale entries remain through administrative down/up cycles. |
| Expected Result | No stale or duplicate entries; clean re-discovery within expected time. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) PASS |

Logs: Configuration and Verification

On DUT-A sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:89:10 sonic fortyGigE0/20 Ethernet20 52:54:00:c3:89:10 sonic fortyGigE0/24 Ethernet24 sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/24 PortDescr: Ethernet24 ----------------------------------------------------------- sonic# ON DUT-B sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- sonic# Admin Down the Interface on DUT-A sonic# configure terminal sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# shutdown sonic(conf-if-Ethernet8)# exit sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# shutdown sonic(conf-if-Ethernet12)# Verify on DUT-B #Ether8 and Ether12 neighbor should show down sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- sonic# # As a result, LLDP frames are not getting received on Ethernet 16 and Ethernet 8 which are connected to DUT-A ON DUT-A sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/20 Ethernet20 52:54:00:c3:89:10 sonic fortyGigE0/24 Ethernet24 sonic# Admin Up the Interface on DUT-A admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# no shutdown sonic(conf-if-Ethernet12)# exit sonic# configure terminal sonic(config)# interface Ethernet 8 sonic(conf-if-Ethernet8)# no shutdown sonic(conf-if-Ethernet8)# Verify on DUT-B #Ether8 and Ether12 neighbor should show UP now. sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic#

#### Verify local Chassis/Port ID subtype correctness

| Test ID | 4.16.24 |
| --- | --- |
| Feature | LLDP |
| Test Case | Verify local Chassis/Port ID subtype correctness |
| Test Item | Functional |
| Test Objective | Confirm local Chassis ID and Port ID subtypes map to configured identifiers/aliases. |
| Expected Result | Subtype and values are correct and stable. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) PASS |

Logs: Configuration and Verification

STEP 1 — Capture Local LLDP (DUT-A) for TLV Verification ON DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- - ChassisID value is a MAC → correct Chassis ID subtype - PortID value is interface name → correct Port ID subtype - PortDescr matches SONiC interface → mapping is correct - Values remain stable → subtype correctness validated STEP 2 — Reboot DUT-A and Confirm Stability After reboot, verify again On DUT-B through show lldp neighbor command Expected: - Same ChassisID - Same PortID - No change in subtype behavior - LLDP neighbor rediscovered correctly sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- This confirms stability of Chassis/Port ID TLVs.

#### Reject invalid LLDP timer values

| Test ID | 4.16.25 |
| --- | --- |
| Feature | LLDP |
| Test Case | Reject invalid LLDP timer values |
| Test Item | Negative |
| Test Objective | Ensure invalid tx-interval/multiplier values are rejected and do not corrupt config. |
| Expected Result | Invalid inputs are rejected; LLDP continues with prior valid settings. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) PASS |

Logs: Configuration and Verification

ON DUT-A STEP 1 — Attempt Invalid LLDP Timer Values Valid Timer range: sonic(config)# lldp timer <5..254> Hello time value (5 to 254 sec) sonic(config)# lldp timer 255 ^ % Error: Invalid input detected at "^" marker. sonic(config)# % Error: Invalid input detected at "^" marker. sonic(config)# lldp timer 1 ^ % Error: Invalid input detected at "^" marker. sonic(config)# Result: % Error: Invalid input detected as Expected. STEP 2 — Attempt Invalid LLDP Multiplier Values Valid Multiplier range: sonic(config)# lldp multiplier <1..10> Multiplier value (1 to 10) sonic(config)# lldp multiplier 11 ^ % Error: Invalid input detected at "^" marker. sonic(config)# lldp multiplier 0 ^ % Error: Invalid input detected at "^" marker. sonic(config)# Result: % Error: Invalid input detected as Expected. STEP 3 — Validate LLDP Operational Stability After injecting invalid values, verify LLDP stays healthy. Check neighbor presence on DUT-B sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- # Neighbor still visible. # No LLDP daemon crash

#### Receive-only with peer receive-only (no discovery)

| Test ID | 4.16.26 |
| --- | --- |
| Feature | LLDP |
| Test Case | Receive-only with peer receive-only (no discovery) |
| Test Item | Negative |
| Test Objective | Demonstrate that Rx-only on both ends yields no neighbors by design. |
| Test Procedure | 1) Set Rx-only on DUT and peer. 2) Verify "show lldp neighbors" remains empty. |
| Expected Result | No neighbors displayed, confirming expected Rx-only behavior. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL-No impact found on either DUT. Both DUT-A and DUT-B continue transmitting LLDP. LLDP RX-only is NOT enforced by SONiC IS-CLI build |

Logs: Configuration and Verification

ON DUT-A admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 12 sonic(conf-if-Ethernet12)# no lldp TRANSMIT sonic(conf-if-Ethernet12)# lldp RECEIVE sonic(conf-if-Ethernet12)# exit sonic# ON DUT-B admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# interface Ethernet 16 sonic(conf-if-Ethernet16)# lldp RECEIVE sonic(conf-if-Ethernet16)# no lldp TRANSMIT sonic(conf-if-Ethernet16)# exit sonic# Verify LLDP neighbor on both DUTs Expected: Both DUTs should not have LLDP neighborship between Ethernet 12(DUT-A) and Ethernet 16(DUT-B) ON DUT-A sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:89:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:89:10 sonic fortyGigE0/20 Ethernet20 52:54:00:c3:89:10 sonic fortyGigE0/24 Ethernet24 sonic# ON DUT-B sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# Result: No impact found on either DUT. Both DUT-A and DUT-B continue transmitting LLDP. LLDP RX-only is NOT enforced by SONiC IS-CLI build.

#### Handle unknown/private TLVs safely

| Test ID | 4.16.27 |
| --- | --- |
| Feature | LLDP |
| Test Case | Handle unknown/private TLVs safely |
| Test Item | Negative |
| Test Objective | Ensure unknown/private TLVs do not crash or corrupt LLDP state. |
| Test Procedure | 1) Peer injects private/unknown TLV. 2) Monitor logs and neighbors for stability. |
| Expected Result | Unknown TLVs are ignored/logged; no crash or state corruption. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) PASS: Unknown TLVs are ignored/logged; no crash or state corruption. |

Logs: Configuration and Verification

STEP 1 — Start tcpdump on DUT-B admin@sonic:~$ sudo tcpdump -i Ethernet16 -vv -s 1500 ether proto 0x88cc tcpdump: listening on Ethernet16, link-type EN10MB (Ethernet), snapshot length 1500 bytes 13:25:31.405052 LLDP, length 262 Chassis ID TLV (1), length 7 Subtype MAC address (4): 52:54:00:c3:89:10 (oui Unknown) 0x0000: 0452 5400 c389 10 Port ID TLV (2), length 14 Subtype Local (7): fortyGigE0/16 0x0000: 0766 6f72 7479 4769 6745 302f 3136 Time to Live TLV (3), length 2: TTL 120s 0x0000: 0078 System Name TLV (5), length 5: sonic 0x0000: 736f 6e69 63 System Description TLV (6), length 142 SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 0x0000: 534f 4e69 4320 536f 6674 7761 7265 2056 0x0010: 6572 7369 6f6e 3a20 534f 4e69 432e 3230 0x0020: 3235 3035 2d73 6d63 692d 6465 762d 6f63 0x0030: 2d6d 322e 302d 3638 3332 3335 3231 3520 0x0040: 2d20 4877 536b 753a 2046 6f72 6365 3130 0x0050: 2d53 3630 3030 202d 2044 6973 7472 6962 0x0060: 7574 696f 6e3a 2044 6562 6961 6e20 3132 0x0070: 2e31 3320 2d20 4b65 726e 656c 3a20 362e 0x0080: 312e 302d 3239 2d32 2d61 6d64 3634 System Capabilities TLV (7), length 4 System Capabilities [Bridge, WLAN AP, Router, Station Only] (0x009c) Enabled Capabilities [Bridge, Router] (0x0014) 0x0000: 009c 0014 Management Address TLV (8), length 12 Management Address length 5, AFI IPv4 (1): 240.127.1.1 Interface Index Interface Numbering (2): 10 0x0000: 0501 f07f 0101 0200 0000 0a00 Management Address TLV (8), length 24 Management Address length 17, AFI IPv6 (2): fd00::1 Interface Index Interface Numbering (2): 10 0x0000: 1102 fd00 0000 0000 0000 0000 0000 0000 0x0010: 0001 0200 0000 0a00 Port Description TLV (4), length 10: Ethernet16 0x0000: 4574 6865 726e 6574 3136 Organization specific TLV (127), length 9: OUI IEEE 802.3 Private (0x00120f) Link aggregation Subtype (3) aggregation status [supported], aggregation port ID 0 0x0000: 0012 0f03 0100 0000 00 Organization specific TLV (127), length 9: OUI IEEE 802.3 Private (0x00120f) MAC/PHY configuration/status Subtype (1) autonegotiation [none] (0x00) These are UNKNOWN TLVs for many platforms. This is evidence of TLV injection. STEP 2 — Observe LLDP Neighbor Table (IS-CLI) sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP --more-- Expected: - Only standard TLVs shown - No garbage/incorrect fields - No malformed TLVs printed - No duplicate neighbor entries #Result: Show lldp neighbor output is stable, standard TLVs intact and Unknown TLVs not shown.

#### Handle malformed LLDP frames (fuzz)

| Test ID | 4.16.28 |
| --- | --- |
| Feature | LLDP |
| Test Case | Handle malformed LLDP frames (fuzz) |
| Test Item | Negative |
| Test Objective | Validate robustness to malformed TLVs/PDUs via traffic generator. |
| Test Procedure | 1) Send malformed LLDP frames (bad lengths/ordering). 2) Observe counters/logs for drops; verify agent stability. |
| Expected Result | Malformed frames are dropped; counters/logs record discards; no agent crash. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) Not Feasible. Could not check in ISCLI |

Logs: Configuration and Verification

#### Ignore VLAN-tagged LLDP frames (policy)

| Test ID | 4.16.29 |
| --- | --- |
| Feature | LLDP |
| Test Case | Ignore VLAN-tagged LLDP frames (policy) |
| Test Item | Negative |
| Test Objective | Ensure VLAN-tagged LLDP PDUs from peer do not create neighbors (deployment policy). |
| Test Procedure | 1) TG sends VLAN-tagged LLDP frames. 2) Observe neighbor table and counters. |
| Expected Result | No neighbors learned from VLAN-tagged LLDP; frames are discarded/ignored. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) Not Feasible as Vlan configuration not working |

Logs: Configuration and Verification

#### Disable LLDP globally with active neighbors

| Test ID | 4.16.30 |
| --- | --- |
| Feature | LLDP |
| Test Case | Disable LLDP globally with active neighbors |
| Test Item | Negative |
| Test Objective | Validate complete withdrawal of neighbors after global disable. |
| Test Procedure | 1) "config lldp global disable". 2) Verify table empties; enable back and verify neighbors return. |
| Expected Result | All neighbors withdrawn on disable; reappear after enable without stale entries. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL-No neighbour withdrawn on disable .Stiall DUT-A neighbors visible in LLDP . |

Logs: Configuration and Verification

Preconditions: Make sure LLDP neighbors exist before disabling LLDP. ON DUT-A sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- Interface: 52:54:00:c3:89:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:89:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/24 PortDescr: Ethernet24 ----------------------------------------------------------- sonic# ON DUT-B sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- sonic# STEP 1 — Disable LLDP globally on DUT-A: sonic# configure terminal sonic(config)# lldp <RECEIVE/TRANSMIT> Configure mode enable Enable LLDP at global level multiplier Configure LLDP multiplier value (1 to 10) system-description Configure LLDP system description system-name Configure LLDP system name timer Configure LLDP hello time value in seconds (5 to 254 sec) tlv-select Enable LLDP TLVs to be advertised sonic(config)# no lldp enable sonic(config)# STEP 2 — Verify LLDP neighbors disappear on DUT-B: sonic# show lldp neighbor ----------------------------------------------------------- LLDP Neighbors ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/4 PortDescr: Ethernet4 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/8 PortDescr: Ethernet8 ----------------------------------------------------------- Interface: 52:54:00:c3:21:31,via: LLDP Chassis: ChassisID: 52:54:00:c3:21:31 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/12 PortDescr: Ethernet12 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/16 PortDescr: Ethernet16 ----------------------------------------------------------- Interface: 52:54:00:c3:47:10,via: LLDP Chassis: ChassisID: 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 TTL: Port PortID: fortyGigE0/20 PortDescr: Ethernet20 ----------------------------------------------------------- sonic# sonic# show lldp table ----------------------------------------------------------------------------------------------------------------------- LocalPort RemoteDevice RemotePortID Capability RemotePortDescr ----------------------------------------------------------------------------------------------------------------------- 52:54:00:c3:21:31 sonic fortyGigE0/4 Ethernet4 52:54:00:c3:47:10 sonic fortyGigE0/8 Ethernet8 52:54:00:c3:21:31 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/12 Ethernet12 52:54:00:c3:47:10 sonic fortyGigE0/16 Ethernet16 52:54:00:c3:47:10 sonic fortyGigE0/20 Ethernet20 sonic# Result: No change observed on LLDP neighbors on DUT B, DUT-A neighbor details still not disappeared.

#### Reject invalid management address TLV config

| Test ID | 4.16.31 |
| --- | --- |
| Feature | LLDP |
| Test Case | Reject invalid management address TLV config |
| Test Item | Negative |
| Test Objective | Ensure malformed Mgmt Address entries are refused. |
| Test Procedure | 1) Configure invalid Mgmt Address value. 2) Verify CLI validation failure; no TLV sent. |
| Expected Result | Invalid configuration rejected; no incorrect TLV observed at peer. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) : PASS |

Logs: Configuration and Verification

On DUT-B Existing LLDP info on interface Ethernet 16 LLDP neighbors: ------------------------------------------------------------------------------- Interface: Ethernet16, via: LLDP, RID: 8, Time: 2 days, 17:16:47 Chassis: ChassisID: mac 52:54:00:c3:47:10 SysName: sonic SysDescr: SONiC Software Version: SONiC.202505-smci-dev-oc-m2.0-683235215 - HwSku: Force10-S6000 - Distribution: Debian 12.13 - Kernel: 6.1.0-29-2-amd64 MgmtIP: 192.168.100.64 MgmtIface: 2 MgmtIP: fd00::1 MgmtIface: 9 Capability: Bridge, on Capability: Router, on Capability: Wlan, off Capability: Station, off Port: PortID: local fortyGigE0/12 PortDescr: Ethernet12 TTL: 120 ------------------------------------------------------------------------------- STEP 1 — Try INVALID Management Address: ON DUT-A sonic(config)# lldp tlv-select management-address sonic(conf-if-Ethernet12)# lldp tlv-set management-address ipv4 1922.168.100.200 ^ % Error: Invalid input detected at "^" marker. sonic(conf-if-Ethernet12)# Result: Malformed ipv4 address rejected as Expected. STEP 2 — Verify config was NOT stored % Error: Invalid input detected at "^" marker. sonic(conf-if-Ethernet12)# exit sonic# show running configuration lldp ^ % Error: Invalid input detected at "^" marker. sonic# show runn # show running configuration lldp not supported. Result: - Error “Invalid input detected” on providing malformed management ip address - NO TLV sent, Management Address unchanged and valid. - No mgmt-address key added or changed. - LLDP functions normally; no malformed TLVs.

#### Per-port disable while peer continues Tx

| Test ID | 4.16.32 |
| --- | --- |
| Feature | LLDP |
| Test Case | Per-port disable while peer continues Tx |
| Test Item | Negative |
| Test Objective | Validate that disabling LLDP on a port hides neighbor and stops Tx. |
| Test Procedure | 1) "config lldp port disable Ethernet64" while peer keeps Tx. 2) Verify neighbor vanishes and Tx ceases. |
| Expected Result | No neighbor shown; Tx halted on disabled port; counters align with behavior. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution)FAIL |

Logs: Configuration and Verification

As per previous tests ,already raised defect for the issues: - no lldp transmit DOES NOT STOP LLDP Tx on your SONiC-VS build - no lldp receive DOES NOT STOP LLDP Rx - Per-port LLDP disable in IS-CLI is NOT functional

#### Duplicate chassis/port-ID collision handling

| Test ID | 4.16.33 |
| --- | --- |
| Feature | LLDP |
| Test Case | Duplicate chassis/port-ID collision handling |
| Test Item | Negative |
| Test Objective | Ensure deterministic behavior when peers advertise identical IDs. |
| Test Procedure | 1) Configure peer to clone chassis/port ID. 2) Monitor neighbor table and logs. |
| Expected Result | No crash; deterministic entry selection (e.g., last update) or multi-entry supported by agent. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution):Not Feasible |

Logs: Configuration and Verification

#### Ensure LLDP not bound to SVIs/subinterfaces

| Test ID | 4.16.34 |
| --- | --- |
| Feature | LLDP |
| Test Case | Ensure LLDP not bound to SVIs/subinterfaces |
| Test Item | Negative |
| Test Objective | Confirm LLDP operates only on physical interfaces. |
| Test Procedure | 1) Create SVI/subinterface. 2) Verify LLDP neighbors are only on physical member ports. |
| Expected Result | No LLDP neighbors on SVIs/subinterfaces; only physical ports participate. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL—Vlan configuration failed |

Logs: Configuration and Verification

ON DUT-A TEP 1 — Create SVI and/or Subinterface ✔ Create a VLAN Interface (SVI) admin@sonic:~$ sonic-cli sonic# configure terminal sonic(config)# vlan 100 ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface vlan100 ^ % Error: Invalid input detected at "^" marker. sonic(config)#

#### Bulk enable LLDP on interface range

| Test ID | 4.16.35 |
| --- | --- |
| Feature | LLDP |
| Test Case | Bulk enable LLDP on interface range |
| Test Item | Range/Bulk |
| Test Objective | Validate enabling LLDP across a range of interfaces in one operation. |
| Test Procedure | 1) "config lldp port enable Ethernet64-Ethernet72". 2) Verify only these ports advertise; others remain unchanged. |
| Expected Result | Presence of neighbors only on enabled range; no unintended ports affected. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) FAIL – Interface Range command not supported |

Logs: Configuration and Verification

sonic(config)# interface range Ethernet sonic(config)# interface range Ethernet 4 -12 ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface range Ethernet 4- 12 ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface range Ethernet 1/1 -2 ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface range sonic(config)# interface range - sonic(config)# interface range - ^ % Error: Invalid input detected at "^" marker. sonic(config)# interface range Ethernet sonic(config)# interface range Ethernet

#### Bulk TLV toggle across range

| Test ID | 4.16.36 |
| --- | --- |
| Feature | LLDP |
| Test Case | Bulk TLV toggle across range |
| Test Item | Range/Bulk |
| Test Objective | Disable or enable specific TLVs across a port range. |
| Test Procedure | 1) "config lldp port tlv disable Ethernet64-Ethernet72 management-address". 2) Verify peer no longer sees Mgmt-Addr for that range; re-enable and confirm. |
| Expected Result | TLV state changes uniformly across the range; no per-port drift. |
| Post-conditions / Cleanup | Remove configurations applied for the test, restore baseline, and confirm no lingering sessions/routes/policies remain. Collect logs if any anomaly observed. |
| Actual Result | Pass / Fail / Partial Pass (to be filled during execution) |

Logs: Configuration and Verification