OSPFv2 Configuration Guide Single-Area ¦ Multi-Area ¦ Redistribution SONiC IS-CLI ¦ Step-by-Step Lab Guide

| Platform | SONiC (D-Link) — IS-CLI |
| --- | --- |
| Protocol | OSPFv2 (IPv4) |
| CLI Mode | router ospf / config-router |
| Applies to | Loopback, Ethernet, VLAN, PortChannel interfaces |

Part 1 — Single-Area OSPFv2 Configuration

## Overview

OSPFv2 (Open Shortest Path First version 2) is a link-state routing protocol for IPv4 networks (RFC 2328). It operates within a single autonomous system, organises routers into areas, and uses the Dijkstra SPF algorithm to compute loop-free shortest paths. Area 0 (backbone area) is mandatory and all other areas must connect to it. This section configures two SONiC routers in a single backbone area.

## Topology

+--------------------+ +--------------------+ ¦ Node 1 ¦ ¦ Node 2 ¦ ¦ (DUT / R1) ¦ ¦ (R2) ¦ ¦ ¦ ¦ ¦ ¦ Lo0: 1.1.1.1/32 ¦ ¦ Lo0: 2.2.2.2/32 ¦ ¦ Eth0: 10.0.0.1/30 +------------+ Eth0: 10.0.0.2/30 ¦ ¦ ¦ Area 0 ¦ ¦ +--------------------+ +--------------------+ <--OSPFv2--> Figure 1 — Single-Area OSPFv2 topology

## Scenario

Both routers form an OSPF adjacency over the Ethernet0 point-to-point link in Area 0. Loopback addresses are advertised into OSPF so each router can reach the other's Router-ID and any associated prefixes. Loopback interfaces are set as passive to suppress unnecessary hellos.

## Node 1 — Configuration (DUT / R1)

| Step 1 | Configure Loopback0 for Router-ID stability |
| --- | --- |

! Create and address the loopback interface configure terminal interface Loopback0 ip address 1.1.1.1/32 no shutdown exit

| Step 2 | Start OSPFv2 process and set Router-ID |
| --- | --- |

! Enter OSPF router configuration mode router ospf ! Assign a stable Router-ID (loopback address) ospf router-id 1.1.1.1

| Step 3 | Advertise networks into Area 0 |
| --- | --- |

! Add the loopback prefix to Area 0 network 1.1.1.1/32 area 0 ! Add the transit link prefix to Area 0 network 10.0.0.0/30 area 0

| Step 4 | Suppress hellos on the loopback (passive interface) |
| --- | --- |

! Loopback does not need OSPF hellos passive-interface Loopback0 exit

| Step 5 | Set the link type to point-to-point on Ethernet0 |
| --- | --- |

! Eliminates DR/BDR election on p2p links configure terminal interface Ethernet0 ip ospf network point-to-point exit

| Step 6 | Verify OSPF neighbor adjacency |
| --- | --- |

show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface 2.2.2.2 0 Full/- 00:00:38 10.0.0.2 Ethernet0

NOTE: State 'Full/-' indicates a point-to-point adjacency (no DR/BDR). If state is stuck at 'Exstart' or 'Exchange', verify MTU and authentication settings match on both sides.

| Step 7 | Verify OSPF routes in the routing table |
| --- | --- |

show ip route ospf

O 2.2.2.2/32 [110/2] via 10.0.0.2, 00:01:14, Ethernet0 O 10.0.0.0/30 is directly connected, Ethernet0

## Node 2 — Configuration (R2)

| Step 1 | Configure Loopback0 for Router-ID stability |
| --- | --- |

configure terminal interface Loopback0 ip address 2.2.2.2/32 no shutdown exit

| Step 2 | Start OSPFv2 process and set Router-ID |
| --- | --- |

router ospf ospf router-id 2.2.2.2 network 2.2.2.2/32 area 0 network 10.0.0.0/30 area 0 passive-interface Loopback0 exit

| Step 3 | Set the link type to point-to-point on Ethernet0 |
| --- | --- |

configure terminal interface Ethernet0 ip ospf network point-to-point exit

NOTE: Both routers must be in the same OSPF area for adjacency to form. Mismatched area types (e.g., one stub, one normal) or mismatched hello/dead timers will prevent the Full state.

Part 2 — Multi-Area OSPFv2 Configuration

## Overview

Multi-area OSPF reduces the size of the link-state database (LSDB) by segmenting the network into areas. Area Border Routers (ABRs) summarise inter-area routes and connect non-backbone areas to Area 0. This section adds a third router (R3) in Area 1, making R2 an ABR between Area 0 and Area 1.

## Topology

+----------+ Area 0 +----------+ Area 1 +----------+ ¦ R1 ¦ ¦ R2 ¦ ¦ R3 ¦ ¦ 1.1.1.1 +----10.0.0.0/30----+ 2.2.2.2 +----10.0.1.0/30--+ 3.3.3.3 ¦ ¦ (Lo0) ¦ Eth0 Eth0 ¦ (ABR) ¦ Eth4 Eth0 ¦ (Lo0) ¦ +----------+ +----------+ +----------+ ¦ Lo0: 2.2.2.2/32 Figure 2 — Multi-area OSPFv2 topology (R2 is the ABR)

## Scenario

R1 and R2 remain in Area 0. R2 is extended with a second Ethernet link toward R3 in Area 1. R2 becomes an ABR and will originate Type-3 Summary LSAs between areas. Inter-area route summarisation is configured on R2 to reduce the number of prefixes propagated.

## Node 2 (R2) — Additional ABR Configuration

| Step 1 | Add the Area 1 link and loopback to OSPF |
| --- | --- |

! Ethernet4 connects R2 to R3 in Area 1 configure terminal interface Ethernet4 ip address 10.0.1.1/30 ip ospf network point-to-point no shutdown exit router ospf network 10.0.1.0/30 area 1 exit

| Step 2 | Configure inter-area route summarisation on the ABR |
| --- | --- |

! Summarise all Area 1 loopbacks (3.x.x.x) into Area 0 router ospf area 1 range 3.0.0.0/8 exit

| Step 3 | Verify OSPF interfaces and areas |
| --- | --- |

show ip ospf interface brief

Interface Area IP Address Cost State Nbrs F/C Loopback0 0 1.1.1.1/32 1 DR 0/0 Ethernet0 0 10.0.0.1/30 1 P2P 1/1 Ethernet4 1 10.0.1.1/30 1 P2P 1/1

## Node 3 — Configuration (R3 in Area 1)

| Step 1 | Configure Loopback0 and Ethernet0, start OSPF |
| --- | --- |

configure terminal interface Loopback0 ip address 3.3.3.3/32 no shutdown exit interface Ethernet0 ip address 10.0.1.2/30 ip ospf network point-to-point no shutdown exit router ospf ospf router-id 3.3.3.3 network 3.3.3.3/32 area 1 network 10.0.1.0/30 area 1 passive-interface Loopback0 exit

| Step 2 | Verify inter-area routes received from ABR |
| --- | --- |

show ip route ospf

O IA 1.1.1.1/32 [110/3] via 10.0.1.1, 00:02:10, Ethernet0 O IA 2.2.2.2/32 [110/2] via 10.0.1.1, 00:02:10, Ethernet0 O IA 10.0.0.0/30 [110/2] via 10.0.1.1, 00:02:10, Ethernet0

NOTE: 'O IA' denotes inter-area OSPF routes (Type-3 Summary LSA). If summarisation is active on R2, only the summary prefix (e.g., 3.0.0.0/8) appears in Area 0 instead of individual /32 entries.

Part 3 — Redistribution & Advanced Features

## Overview

OSPF can redistribute routes from other protocols (connected, static, BGP) into the OSPF domain as Type-5 External LSAs (E1 or E2). Authentication can be enabled per-interface or per-area to secure adjacencies. Default route origination allows OSPF to advertise a default gateway to all routers.

## Redistribution into OSPF

| Step 1 | Redistribute connected and static routes into OSPF |
| --- | --- |

! On R1 — redistribute all connected interfaces router ospf redistribute connected metric 10 metric-type 2 ! On R1 — redistribute static routes redistribute static metric 20 metric-type 1 exit

| Step 2 | Originate a default route into OSPF |
| --- | --- |

! Inject default route — only advertised if R1 has one router ospf default-information originate always metric 5 exit

| Step 3 | Verify external routes on R2 |
| --- | --- |

show ip route ospf

O E2 0.0.0.0/0 [110/5] via 10.0.0.1, Ethernet0 O E2 192.168.1.0/24 [110/10] via 10.0.0.1, Ethernet0 O E1 172.16.0.0/12 [110/21] via 10.0.0.1, Ethernet0

NOTE: E1 routes include the internal OSPF cost to the ASBR; E2 routes use only the external metric. E1 is preferred over E2 when both exist for the same prefix.

## MD5 Authentication

| Step 1 | Enable MD5 authentication on the transit interface |
| --- | --- |

! Configure on both R1 and R2 — keys must match configure terminal interface Ethernet0 ip ospf authentication message-digest ip ospf message-digest-key 1 md5 P@lcN3tw0rk! exit

| Step 2 | Verify authentication is active |
| --- | --- |

show ip ospf interface Ethernet0

Ethernet0 is up, line protocol is up Internet address is 10.0.0.1/30, Area 0 Process ID 1, Router ID 1.1.1.1, Network type POINT_TO_POINT Authentication: Message digest Youngest key id: 1 Timer intervals: Hello 10s, Dead 40s, Retransmit 5s Neighbor Count is 1

NOTE: Mismatched authentication type or key causes adjacency to drop. Use 'debug ip ospf adj' to diagnose authentication failures. Always configure authentication on both ends before applying.

Part 4 — Quick Reference

## OSPFv2 — Common show commands

| Goal | IS-CLI Command |
| --- | --- |
| View OSPF neighbor table | show ip ospf neighbor |
| View OSPF database (all LSAs) | show ip ospf database |
| View OSPF routes in RIB | show ip route ospf |
| Check OSPF interface settings | show ip ospf interface |
| View OSPF process summary | show ip ospf |
| Check OSPF border routers | show ip ospf border-routers |
| View Type-3 summary LSAs | show ip ospf database summary |
| View Type-5 external LSAs | show ip ospf database external |
| Check OSPF statistics | show ip ospf statistics |
| Debug adjacency events | debug ip ospf adj |

## OSPFv2 — Configuration command reference

| Goal | IS-CLI Command |
| --- | --- |
| Set Router-ID | ospf router-id A.B.C.D |
| Add network to area | network A.B.C.D/prefix area <id> |
| Suppress hellos on interface | passive-interface <intf> |
| Set interface as point-to-point | ip ospf network point-to-point |
| Set interface as broadcast | ip ospf network broadcast |
| Change hello interval | ip ospf hello-interval <sec> |
| Change dead interval | ip ospf dead-interval <sec> |
| Set OSPF cost on interface | ip ospf cost <1-65535> |
| Summarise area prefix (ABR) | area <id> range A.B.C.D/prefix |
| Define stub area | area <id> stub |
| Define NSSA area | area <id> nssa |
| Enable MD5 auth on interface | ip ospf authentication message-digest |
| Set MD5 key on interface | ip ospf message-digest-key <id> md5 <key> |
| Redistribute connected | redistribute connected metric <n> metric-type <1¦2> |
| Redistribute static | redistribute static metric <n> metric-type <1¦2> |
| Originate default route | default-information originate [always] metric <n> |
| Remove network from OSPF | no network A.B.C.D/prefix area <id> |
| Shutdown OSPF process | no router ospf |

## Binding / Adjacency cheat-sheet

| Scenario | Key Setting | Command |
| --- | --- | --- |
| P2P link (no DR/BDR) | Network type | ip ospf network point-to-point |
| LAN segment (DR election) | Network type | ip ospf network broadcast |
| Quiet loopback | Passive | passive-interface Loopback0 |
| Fast convergence | Timers | ip ospf hello-interval 1 / dead-interval 4 |
| Influence DR election | Priority | ip ospf priority <0-255> |
| Stub area (no externals) | Area type | area <id> stub |
| NSSA (local ASBR in stub) | Area type | area <id> nssa |
| Secure adjacency | Authentication | ip ospf authentication message-digest |