VLAN Configuration Guide Access Ports ¦ Trunk Ports ¦ Mixed Mode SONiC IS-CLI ¦ Step-by-Step Lab Guide

| Platform | SONiC (D-Link) — IS-CLI |
| --- | --- |

| VLAN Range | VLAN 1 – 4094 (IEEE 802.1Q) |
| --- | --- |

| CLI Mode | config / interface / vlan |
| --- | --- |

| Applies to | Ethernet, PortChannel, VLAN SVI interfaces |
| --- | --- |

Part 1 — VLAN Creation & Deletion

## Overview

A VLAN (Virtual Local Area Network) segments a physical network into independent broadcast domains at Layer 2. SONiC supports VLAN IDs 1–4094 per IEEE 802.1Q. VLANs must be created before assigning ports. VLAN 1 is the system default and exists on all ports as the native VLAN unless explicitly reconfigured. VLANs are configured using the IS-CLI interface and take effect immediately in the running configuration.

## Topology

+-----------------------------+ +-----------------------------+ ¦ SONiC-VS (Spine/Partner) ¦ ¦ DUT (Leaf — SONiC) ¦ ¦ ¦ ¦ ¦ ¦ Eth0 ─────────────────────── Port1 (Access VLAN 10) ¦ ¦ Eth1 ─────────────────────── Port2 (Access VLAN 20) ¦ ¦ Eth2 ─────────────────────── Port3 (Trunk 10,20,30) ¦ ¦ Eth3 ─────────────────────── Port4 (Mixed: T10/U20) ¦ ¦ Eth4 ─────────────────────── Port5 (Trunk 10,20,30) ¦ ¦ Scapy TX/RX ¦ ¦ ¦ +-----------------------------+ +-----------------------------+ Figure 1 — Leaf-spine test topology (5 ports between DUT and SONiC-VS)

## Scenario

Create VLANs, verify their existence in the running configuration, and test deletion. The valid VLAN range is 1–4094; IDs 0 and 4095 are reserved and will be rejected by the system.

## Node 1 (DUT) — VLAN Creation

| Step 1 | Create a single VLAN |
| --- | --- |

! Enter global configuration mode configure terminal ! Create VLAN 10 vlan 10 exit

| Step 2 | Create multiple VLANs at once |
| --- | --- |

! Create a range of VLANs in one command configure terminal vlan 10,20,30,40,50 exit

| Step 3 | Verify VLAN creation |
| --- | --- |

show vlan

Q: A - Access (Untagged), T - Tagged NUM Status Q Ports Autostate Dynamic Vlan10 Active Enable No Vlan20 Active Enable No Vlan30 Active Enable No Vlan40 Active Enable No Vlan50 Active Enable No

| Step 4 | Delete a VLAN |
| --- | --- |

! Remove VLAN 50 from the database configure terminal no vlan 50 exit

| Step 5 | Verify running configuration |
| --- | --- |

show running-config

NOTE: Attempting to create VLAN 0 or VLAN 4095 returns an error: '% Invalid input detected'. Attempting to delete VLAN 1 (default VLAN) may be rejected or have limited effect depending on the platform. Always verify with 'show running-config' after bulk operations.

Part 2 — Access Port Configuration

## Overview

An access port belongs to exactly one untagged VLAN. Frames arriving on an access port are internally tagged with the configured VLAN ID and stripped of that tag on egress. Access ports are typically used for end-device connections (servers, workstations, printers) that send untagged traffic. Packets entering an access port are forwarded only within the same VLAN, ensuring Layer-2 isolation between devices on different VLANs.

## Scenario

Configure Port1 as an access port in VLAN 10 and Port2 in VLAN 20. Verify traffic isolation — a frame from Port1 (VLAN 10) must not reach Port2 (VLAN 20). Then place both ports in the same VLAN and confirm successful forwarding.

## Node 1 (DUT) — Access Port Configuration

| Step 1 | Create VLANs and assign access ports |
| --- | --- |

configure terminal vlan 10 vlan 20 exit ! Assign Port1 as untagged member of VLAN 10 interface Ethernet1 switchport mode access switchport access vlan 10 exit ! Assign Port2 as untagged member of VLAN 20 interface Ethernet2 switchport mode access switchport access vlan 20 exit

| Step 2 | Verify access port configuration |
| --- | --- |

show vlan

Q: A - Access (Untagged), T - Tagged NUM Status Q Ports Autostate Dynamic Vlan10 Active A Ethernet1 Enable No Vlan20 Active A Ethernet2 Enable No

| Step 3 | Change access port VLAN membership |
| --- | --- |

! Move Port1 from VLAN 10 to VLAN 20 configure terminal interface Ethernet1 switchport access vlan 20 exit

## Node 2 (SONiC-VS) — Traffic Generation

| Step 1 | Send untagged frames using Scapy and verify isolation |
| --- | --- |

# vim scapy_access.py from scapy.all import * # Test 1 — send from Port1 (VLAN 10) toward Port2 MAC (VLAN 20) — should be DROPPED pkt1 = Ether(src='00:11:22:33:44:01', dst='00:11:22:33:44:02') sendp(pkt1, iface='Eth0', count=50) # Eth0 connects to Port1 # Test 2 — send within VLAN 10 — should be FORWARDED pkt2 = Ether(src='00:11:22:33:44:01', dst='ff:ff:ff:ff:ff:ff') sendp(pkt2, iface='Eth0', count=50)

NOTE: VLAN isolation: frames entering Port1 (VLAN 10) are never forwarded to Port2 (VLAN 20) without a Layer-3 gateway performing inter-VLAN routing. Use 'show interfaces counters -i Ethernet1' on the DUT to verify RX_DRP counts.

Part 3 — Trunk Port Configuration

## Overview

A trunk port carries tagged frames for multiple VLANs simultaneously. The VLAN tag (802.1Q header) is preserved across the trunk link so the receiving device knows which VLAN each frame belongs to. Trunk ports are used for switch-to-switch uplinks, router-on-a-stick connections, and server NICs that process VLAN-tagged traffic. Frames for VLANs not in the trunk's allowed list are dropped.

## Scenario

Configure Port3 and Port5 as trunk ports carrying VLANs 10, 20, and 30. Send tagged frames on each VLAN and confirm forwarding. Attempt to send a VLAN 40 tagged frame on Port3 (not in the allowed list) and confirm it is dropped. Then dynamically add and remove VLANs from the trunk.

## Node 1 (DUT) — Trunk Port Configuration

| Step 1 | Create VLANs and configure trunk ports |
| --- | --- |

configure terminal vlan 10,20,30 exit ! Configure Port3 as a trunk port interface Ethernet3 switchport mode trunk switchport trunk allowed vlan 10,20,30 exit ! Configure Port5 as a trunk port interface Ethernet5 switchport mode trunk switchport trunk allowed vlan 10,20,30 exit

| Step 2 | Verify trunk port VLAN membership |
| --- | --- |

show vlan

Q: A - Access (Untagged), T - Tagged NUM Status Q Ports Autostate Dynamic Vlan10 Active T Ethernet3 Enable No T Ethernet5 Vlan20 Active T Ethernet3 Enable No T Ethernet5 Vlan30 Active T Ethernet3 Enable No T Ethernet5

| Step 3 | Add a VLAN to an existing trunk dynamically |
| --- | --- |

! Add VLAN 40 to Port3 trunk without disruption configure terminal vlan 40 exit interface Ethernet3 switchport trunk allowed vlan add 40 exit

| Step 4 | Remove a VLAN from the trunk |
| --- | --- |

! Remove VLAN 30 from Port3 allowed list configure terminal interface Ethernet3 switchport trunk allowed vlan remove 30 exit

| Step 5 | Verify final trunk configuration |
| --- | --- |

show running-config interface Ethernet3

interface Ethernet3 switchport mode trunk switchport trunk allowed vlan 10,20,40

## Node 2 (SONiC-VS) — Traffic Generation

| Step 1 | Send tagged frames across the trunk and verify VLAN filtering |
| --- | --- |

from scapy.all import * # Test 1 — VLAN 10 tagged frame (allowed) — should be FORWARDED pkt_v10 = Ether()/Dot1Q(vlan=10)/IP(src='10.0.10.1',dst='10.0.10.2') sendp(pkt_v10, iface='Eth2', count=50) # Test 2 — VLAN 30 tagged frame on Port3 (removed) — should be DROPPED pkt_v30 = Ether()/Dot1Q(vlan=30)/IP(src='10.0.30.1',dst='10.0.30.2') sendp(pkt_v30, iface='Eth2', count=50)

NOTE: Trunk ports drop frames tagged with VLANs not in their allowed list. Use 'switchport trunk allowed vlan all' to permit all VLANs, or 'switchport trunk allowed vlan none' to block all VLANs and then selectively add. The 'except' keyword (e.g., 'allowed vlan except 100') excludes specific VLANs.

Part 4 — Mixed Port Configuration (Tagged + Untagged)

## Overview

A mixed-mode port carries both tagged traffic for one or more VLANs and untagged traffic for a native (access) VLAN. Untagged ingress frames are assigned to the native VLAN; tagged ingress frames are placed into their respective VLANs. On egress, frames for the native VLAN are sent untagged, while frames for other VLANs are sent with their 802.1Q tag preserved. This is common for IP phones (tagged voice VLAN) + PC (untagged data VLAN) behind a single port.

## Scenario

Configure Port4 with VLAN 20 as the native (untagged) VLAN and VLAN 10 as a tagged VLAN. Send an untagged frame and confirm it is handled as VLAN 20. Send a VLAN 10 tagged frame and confirm it is forwarded within VLAN 10.

## Node 1 (DUT) — Mixed Port Configuration

| Step 1 | Create VLANs and configure Port4 as mixed mode |
| --- | --- |

configure terminal vlan 10,20 exit interface Ethernet4 ! Set the native (untagged) VLAN to 20 switchport mode trunk switchport access vlan 20 ! Allow tagged VLAN 10 in addition switchport trunk allowed vlan 10 exit

| Step 2 | Verify mixed port configuration |
| --- | --- |

show running-config interface Ethernet4

interface Ethernet4 switchport mode trunk switchport access vlan 20 switchport trunk allowed vlan 10

| Step 3 | Verify VLAN membership table |
| --- | --- |

show vlan

Q: A - Access (Untagged), T - Tagged NUM Status Q Ports Autostate Dynamic Vlan10 Active T Ethernet4 Enable No Vlan20 Active A Ethernet4 Enable No

## Node 2 (SONiC-VS) — Traffic Generation

| Step 1 | Test untagged and tagged traffic on the mixed port |
| --- | --- |

from scapy.all import * # Test 1 — send untagged frame — DUT assigns it to native VLAN 20 untagged = Ether(src='aa:bb:cc:00:00:01', dst='ff:ff:ff:ff:ff:ff') sendp(untagged, iface='Eth3', count=20) # Eth3 connects to Port4 # Test 2 — send VLAN 10 tagged frame — forwarded within VLAN 10 tagged = Ether()/Dot1Q(vlan=10)/IP(src='10.0.10.1',dst='10.0.10.2') sendp(tagged, iface='Eth3', count=20)

NOTE: Q-in-Q (double-tagged / 802.1ad) frames arrive with two VLAN tags (outer + inner). The switch processes the outer tag; inner tags are passed transparently. To match double-tagged frames in Scapy: Ether()/Dot1Q(vlan=10)/Dot1Q(vlan=20)/IP(...)

Part 5 — Packet Forwarding & Tagging Behaviour

## Overview

VLAN forwarding ensures that unicast, broadcast, and multicast frames are delivered only to ports belonging to the same VLAN. Unknown unicast frames (destination MAC not yet in the MAC table) are flooded to all ports in the VLAN except the ingress port. Tagged frames arriving on trunk ports retain their tags on trunk egress but are stripped when egressing an access port in the same VLAN.

## Verification — show commands

| Step 1 | Check interface counters after traffic generation |
| --- | --- |

! Verify RX / TX counts and drop counters on DUT show interfaces counters -i Ethernet1 show interfaces counters -i Ethernet3

IFACE STATE RX_OK RX_BPS RX_ERR RX_DRP TX_OK TX_BPS --------- ----- ------ ---------- ------ ------ ------ ---------- Ethernet1 U 100 1.20 KB/s 0 50 50 0.60 KB/s Ethernet3 U 200 2.40 KB/s 0 0 200 2.40 KB/s RX_DRP=50 (frames for non-member VLAN dropped by isolation rule)

| Step 2 | Verify MAC address table per VLAN |
| --- | --- |

show mac address-table show mac address-table vlan 10

VlanId MacAddress Type Ports ------ ----------------- ------- --------- 10 aa:bb:cc:00:00:01 Dynamic Ethernet1 10 aa:bb:cc:00:00:03 Dynamic Ethernet3 20 aa:bb:cc:00:00:02 Dynamic Ethernet2

NOTE: Unknown unicast flooding: when the destination MAC is not in the MAC table, the frame is flooded to all member ports of that VLAN except the ingress port. Once the destination replies, its MAC is learned and subsequent frames are forwarded directly. Use 'clear mac address-table dynamic' to flush the MAC table for testing.

Part 6 — Test Plan Summary (58 Test Cases)

## Overview

This section summarises the complete VLAN test plan comprising 58 test cases across 11 functional areas. All test cases use Scapy-based packet generation and the SONiC IS-CLI for configuration and verification. The topology consists of 1 DUT (leaf SONiC switch) and 1 SONiC-VS system (spine/partner) connected via 5 test ports. Total estimated execution time is 35 hours.

## Key Test Metrics

| Metric | Target |
| --- | --- |

| Maximum VLANs Tested | 4094 (full range, VLAN 1–4094) |
| --- | --- |

| Maximum VLANs per Port | 4093 (all VLANs on a single trunk port) |
| --- | --- |

| Maximum MAC Addresses per VLAN | 1000+ |
| --- | --- |

| Maximum Total MAC Addresses | 10,000 |
| --- | --- |

| Maximum Packet Rate | 10,000 pps |
| --- | --- |

| Long-Duration Stability Test | 24 hours continuous traffic |
| --- | --- |

| Acceptable Packet Loss (Functional) | < 5% |
| --- | --- |

| Acceptable Packet Loss (Scaling) | < 2% |
| --- | --- |

| CPU Usage Threshold | < 80% under load |
| --- | --- |

| Bulk VLAN Config Time | < 5 minutes for 1,000 VLANs |
| --- | --- |

## Test Case Register

| Test Case ID | Objective | Key Steps / CLI | Expected Result | Category |
| --- | --- | --- | --- | --- |

| TC_VLAN_CREATE_001 | Create single VLAN | vlan 10; show running-config | VLAN 10 created, visible in config | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_CREATE_002 | Create multiple VLANs | vlan 10,20,30,40,50; verify all | All VLANs created successfully | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_CREATE_003 | Valid range (1 and 4094) | vlan 1; vlan 4094; verify | Both boundary VLANs created | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_CREATE_004 | Invalid range (0, 4095) | vlan 0; vlan 4095; check error | System rejects with error message | Error |
| --- | --- | --- | --- | --- |

| TC_VLAN_DELETE_001 | Delete single VLAN | vlan 100; no vlan 100; verify | VLAN 100 removed from config | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_DELETE_002 | Delete VLAN with members | Add port; attempt no vlan 100 | System handles gracefully | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_ACCESS_001 | Configure untagged port | switchport access vlan 10; send untagged | Port assigned; ingress tagged VLAN 10 | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_ACCESS_002 | Access port traffic isolation | Port1=VLAN10, Port2=VLAN20; send P1→P2 | Frame NOT received on Port2 | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_ACCESS_003 | Same-VLAN communication | Port1=Port2=VLAN10; send P1→P2 MAC | Frame received on Port2 | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_ACCESS_004 | Change access VLAN membership | Move Port1 from VLAN10 to VLAN20 | Port membership changes cleanly | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_TRUNK_001 | Configure tagged trunk port | switchport mode trunk; allowed vlan 10,20,30 | Port carries all 3 VLANs tagged | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_TRUNK_002 | Multi-VLAN trunk traffic | Send VLAN10/20 frames; verify access egress | Correct VLAN delivered untagged | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_TRUNK_003 | Trunk VLAN filtering | Send VLAN30 (not allowed) on trunk | Frame dropped; not forwarded | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_TRUNK_004 | Add/remove VLANs from trunk | allowed vlan add 40; remove 10; verify | Config updated dynamically | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_MIXED_001 | Tagged + untagged on same port | access vlan 20; trunk allowed 10 | Port shows both T and A in show vlan | Functional |
| --- | --- | --- | --- | --- |

| TC_VLAN_MIXED_002 | Mixed port traffic handling | Send untagged and VLAN10 tagged frames | Correct VLAN assignment for each | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_MIXED_003 | Native VLAN behaviour | Untagged ingress on trunk port | Assigned to native VLAN, forwarded | Tagging |
| --- | --- | --- | --- | --- |

| TC_VLAN_FORWARD_001 | Intra-VLAN unicast | Send unicast Port1→Port2 in VLAN10 | Received on Port2 only | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_FORWARD_002 | Intra-VLAN broadcast | Send broadcast from Port1 in VLAN10 | Received on Port2, Port3; not others | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_FORWARD_003 | Intra-VLAN multicast | Send multicast from Port1 in VLAN10 | Received on all VLAN10 members | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_FORWARD_004 | Inter-VLAN isolation | VLAN10→VLAN20 without routing | Frame NOT forwarded across VLANs | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_FORWARD_005 | Unknown unicast flooding | Send to unknown MAC in VLAN10 | Flooded to all VLAN10 ports | Forwarding |
| --- | --- | --- | --- | --- |

| TC_VLAN_TAG_001 | Ingress untagged tagging | Untagged in on access; capture on trunk | Trunk egress has VLAN10 tag | Tagging |
| --- | --- | --- | --- | --- |

| TC_VLAN_TAG_002 | Egress untagging on access port | Tagged in on trunk; capture on access | Access egress is untagged | Tagging |
| --- | --- | --- | --- | --- |

| TC_VLAN_TAG_003 | Tag preserved on trunk egress | Tagged in on trunk; capture on trunk | VLAN10 tag present on egress | Tagging |
| --- | --- | --- | --- | --- |

| TC_VLAN_TAG_004 | Double-tagged (Q-in-Q) handling | Send outer=10/inner=20 double-tagged | Handled per switch policy | Tagging |
| --- | --- | --- | --- | --- |

| TC_VLAN_PERSIST_001 | Config save and reload | Create VLANs; config save; reload | All VLANs persist after reload | Persistence |
| --- | --- | --- | --- | --- |

| TC_VLAN_PERSIST_002 | Running-config accuracy | Create VLAN; add ports; show running-config | Config accurately reflects state | Persistence |
| --- | --- | --- | --- | --- |

| TC_VLAN_EDGE_001 | Maximum VLANs (4094) | Create VLANs 1–4094 via script | All created; system stable | Edge |
| --- | --- | --- | --- | --- |

| TC_VLAN_EDGE_002 | All ports in single VLAN | Add all 5 ports to VLAN100; test forwarding | All ports communicate in VLAN100 | Edge |
| --- | --- | --- | --- | --- |

| TC_VLAN_EDGE_003 | Rapid create/delete (100x) | Create/delete VLAN 100 x100 iterations | System remains stable throughout | Edge |
| --- | --- | --- | --- | --- |

| TC_VLAN_EDGE_004 | VLAN 1 special handling | Test add/remove on VLAN1; attempt delete | VLAN1 handled per spec | Edge |
| --- | --- | --- | --- | --- |

| TC_VLAN_EDGE_005 | Port in maximum VLANs | Add Port3 to all VLANs as trunk | Port functions with all memberships | Edge |
| --- | --- | --- | --- | --- |

| TC_VLAN_ERROR_001 | Add port to non-existent VLAN | switchport access vlan 999 (not created) | Error message; config unchanged | Error |
| --- | --- | --- | --- | --- |

| TC_VLAN_ERROR_002 | Duplicate VLAN creation | Create VLAN10 twice; verify response | Handled gracefully (idempotent) | Error |
| --- | --- | --- | --- | --- |

| TC_VLAN_ERROR_003 | Invalid VLAN ID values | vlan 5000; vlan -1; vlan abc | All rejected with clear errors | Error |
| --- | --- | --- | --- | --- |

| TC_VLAN_PERF_001 | High traffic in single VLAN | 1000 pps on VLAN10; monitor loss | Forwarding accurate; loss < 5% | Performance |
| --- | --- | --- | --- | --- |

| TC_VLAN_PERF_002 | Multi-VLAN simultaneous traffic | 5 VLANs simultaneous Scapy traffic | All VLANs forwarded; no mixing | Performance |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_001 | Maximum VLAN creation (4094) | Scripted create 1–4094; verify all; reload | All 4094 VLANs created and persist | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_002 | Maximum ports per VLAN | Add all 5 ports to VLAN100; broadcast test | Broadcast reaches all ports | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_003 | Max VLANs per trunk port (4093) | Port3 tagged in VLANs 2–4094; verify config | Port handles all memberships | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_004 | Large MAC table (1000+ per VLAN) | Send 1000 unique src MACs; verify learning | MACs learned; unicast forwarded | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_005 | High pps across 10 VLANs (10k pps) | Simultaneous 10k pps; monitor CPU/loss | < 1% loss; CPU < 80% | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_006 | Bulk config time (1000 VLANs) | Create VLAN 100–1099; measure time | Completes < 5 minutes | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_007 | Bulk port membership config | Port3 added to 491 VLANs; measure time | Config completes efficiently | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_008 | Memory usage at scale | Create 1000 VLANs in batches; monitor RAM | Linear growth; no leaks | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_009 | CPU usage at scale | 1000 VLANs + high-rate traffic; monitor CPU | CPU < 80% throughout | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_010 | VLAN DB consistency (2000 VLANs) | Create 2000; save; reload; compare config | Configs match exactly | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_011 | Broadcast storm at scale (91 VLANs) | 100 pps broadcast per VLAN simultaneously | Contained per VLAN; no storms | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_012 | Rapid membership changes (1000x) | Port1: 1000 add/remove/change iterations | Stable; final config correct | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_013 | Mixed traffic (UC/BC/MC) at scale | 41 VLANs; 5000 pps mixed traffic types | < 2% loss; VLAN isolation intact | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_014 | Forwarding table scale (10k MACs) | 100 VLANs x 100 MACs; unicast verify | Correct forwarding at scale | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_015 | Config rollback at scale (500 VLANs) | Checkpoint 500; add 500 more; rollback | Returns to 500-VLAN baseline | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_016 | Inter-VLAN routing scale (50 VLANs) | 50 VLANs with IP SVI; routed traffic | Routing scales; acceptable perf | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_017 | Bulk VLAN deletion (1000 VLANs) | Delete VLAN 1000–1999 via script; measure | Deletion completes; system stable | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_018 | Concurrent VLAN operations | 4 sessions: create/config/traffic concurrently | No conflicts; all ops succeed | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_019 | 24-hour stability test | 100 VLANs; 1000 pps; 24h continuous | No drift; CPU/RAM stable | Scaling |
| --- | --- | --- | --- | --- |

| TC_VLAN_SCALE_020 | Resource exhaustion recovery | Fill to 4094; attempt +1; delete 10; recreate | Error on limit; recovers cleanly | Scaling |
| --- | --- | --- | --- | --- |

## Execution Schedule

| Phase | Test Cases | Estimated Duration |
| --- | --- | --- |

| Phase 1 | VLAN Creation/Deletion (TC_VLAN_CREATE_*, TC_VLAN_DELETE_*) | 2 hours |
| --- | --- | --- |

| Phase 2 | Access Port Configuration (TC_VLAN_ACCESS_*) | 3 hours |
| --- | --- | --- |

| Phase 3 | Trunk Port Configuration (TC_VLAN_TRUNK_*) | 3 hours |
| --- | --- | --- |

| Phase 4 | Mixed Port Configuration (TC_VLAN_MIXED_*) | 2 hours |
| --- | --- | --- |

| Phase 5 | Packet Forwarding (TC_VLAN_FORWARD_*) | 4 hours |
| --- | --- | --- |

| Phase 6 | Tagging / Untagging (TC_VLAN_TAG_*) | 3 hours |
| --- | --- | --- |

| Phase 7 | Configuration Persistence (TC_VLAN_PERSIST_*) | 2 hours |
| --- | --- | --- |

| Phase 8 | Edge Cases (TC_VLAN_EDGE_*) | 3 hours |
| --- | --- | --- |

| Phase 9 | Error Handling (TC_VLAN_ERROR_*) | 2 hours |
| --- | --- | --- |

| Phase 10 | Performance Tests (TC_VLAN_PERF_*) | 3 hours |
| --- | --- | --- |

| Phase 11 | Scaling Tests (TC_VLAN_SCALE_001 – 020) | 8 hours |
| --- | --- | --- |

| TOTAL |  | 35 hours |
| --- | --- | --- |

Part 7 — Quick Reference

## VLAN Management — Common commands

| Goal | IS-CLI Command |
| --- | --- |

| Create a single VLAN | vlan <id> |
| --- | --- |

| Create multiple VLANs at once | vlan 10,20,30,40 |
| --- | --- |

| Create a range of VLANs | vlan 100-200 |
| --- | --- |

| Delete a VLAN | no vlan <id> |
| --- | --- |

| Delete a range of VLANs | no vlan 100-200 |
| --- | --- |

| Show VLAN database and port membership | show vlan |
| --- | --- |

| Show a specific VLAN | show vlan id <id> |
| --- | --- |

| Show running configuration | show running-config |
| --- | --- |

| Verify interface VLAN assignment | show running-config interface <intf> |
| --- | --- |

| Show MAC address table | show mac address-table |
| --- | --- |

| Show MAC table for specific VLAN | show mac address-table vlan <id> |
| --- | --- |

| Clear dynamic MAC entries | clear mac address-table dynamic |
| --- | --- |

| Show interface counters | show interfaces counters -i <intf> |
| --- | --- |

## Port Mode Configuration — command reference

| Goal | IS-CLI Command |
| --- | --- |

| Set port to access mode | switchport mode access |
| --- | --- |

| Assign access VLAN | switchport access vlan <id> |
| --- | --- |

| Set port to trunk mode | switchport mode trunk |
| --- | --- |

| Allow specific VLANs on trunk | switchport trunk allowed vlan <list> |
| --- | --- |

| Add VLAN to existing trunk | switchport trunk allowed vlan add <id> |
| --- | --- |

| Remove VLAN from trunk | switchport trunk allowed vlan remove <id> |
| --- | --- |

| Allow all VLANs on trunk | switchport trunk allowed vlan all |
| --- | --- |

| Remove all VLANs from trunk | switchport trunk allowed vlan none |
| --- | --- |

| Exclude specific VLANs from trunk | switchport trunk allowed vlan except <id> |
| --- | --- |

| Set native VLAN on trunk port | switchport access vlan <id> (on trunk) |
| --- | --- |

| Remove port from VLAN (access) | no switchport access vlan |
| --- | --- |

| Reset port to default VLAN config | no switchport mode |
| --- | --- |

## Port Configuration Cheat-sheet

| Port Type | Use Case | Key Commands |
| --- | --- | --- |

| Access | End devices (PC, printer, server) | switchport mode access; switchport access vlan <id> |
| --- | --- | --- |

| Trunk | Switch uplinks, router-on-a-stick | switchport mode trunk; switchport trunk allowed vlan <list> |
| --- | --- | --- |

| Mixed (Hybrid) | IP phone + PC behind single port | switchport mode trunk; access vlan <native>; trunk allowed <voice> |
| --- | --- | --- |

| Trunk — all VLANs | Uplink carrying all VLANs | switchport mode trunk; switchport trunk allowed vlan all |
| --- | --- | --- |

| Trunk — excluded | Block specific VLANs on trunk | switchport trunk allowed vlan except <id> |
| --- | --- | --- |