NTP Configuration Guide Authentication ¦ Source Interface ¦ VRF SONiC IS-CLI ¦ Step-by-Step Lab Guide

| Platform | SONiC (D-Link) — IS-CLI |
| --- | --- |
| Protocol | NTP v3 / v4 (RFC 5905) |
| CLI Mode | configure / config-router |
| Auth Types | md5 ¦ sha1 ¦ sha256 ¦ sha384 ¦ sha512 |
| Applies to | Management, Ethernet, Loopback, PortChannel, VLAN interfaces |

Part 1 — NTP Enable & Basic Configuration

## Overview

NTP (Network Time Protocol) synchronises the system clock of a SONiC device with one or more external time sources. The NTP service is disabled by default; it must be administratively enabled before any synchronisation can occur. Disabling NTP stops the daemon but preserves all server and authentication configuration so the service can be re-enabled without reconfiguration. NTP uses UDP port 123 and supports versions 3 and 4.

## Topology

+--------------------+ +--------------------+ +------------------+ ¦ DUT1 ¦ ¦ DUT2 ¦ ¦ NTP-SRV ¦ ¦ (SONiC Client) ¦ ¦ (SONiC Client) ¦ ¦ (Linux / chrony) ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ Mgmt0:192.168.100.1+----------+Mgmt0:192.168.100.2+----------+eth0:192.168.100.10¦ ¦ Eth0: 10.0.0.1/30 +----------+Eth0: 10.0.0.2/30 ¦ ¦ UDP/123 NTPv4 ¦ ¦ Lo0: 1.1.1.1/32 ¦ ¦ ¦ ¦ Keys: MD5/SHA* ¦ +--------------------+ +--------------------+ +------------------+ ¦_______________ Management Network 192.168.100.0/24 _______________¦ Figure 1 — NTP lab topology: 2 DUT nodes + 1 NTP server

## NTP-SRV Setup (chrony)

Before running any test, configure chronyd on NTP-SRV with all required authentication keys. Edit /etc/chrony.conf and /etc/chrony.keys as shown below, then restart the service.

| Step 1 | Configure chrony on NTP-SRV |
| --- | --- |

# /etc/chrony.conf local stratum 2 allow 192.168.100.0/24 keyfile /etc/chrony.keys # /etc/chrony.keys (key-id algorithm secret) 1 MD5 MySecret123 2 SHA256 SecurePass456 3 SHA1 Sha1Password 4 SHA512 BigSecret789 5 SHA384 MediumSecret # Restart chrony sudo systemctl restart chronyd sudo systemctl status chronyd

## DUT1 — Enable and Disable NTP

| Step 1 | Enable NTP service |
| --- | --- |

! NTP is disabled by default configure terminal ntp enable exit

| Step 2 | Verify NTP is enabled |
| --- | --- |

show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: -

| Step 3 | Disable NTP service |
| --- | --- |

! Stops the daemon; all server/auth config is preserved configure terminal no ntp enable exit show ntp global

NTP Configuration: Enabled: False Authentication: False Vrf: default Source Interface: -

NOTE: 'no ntp enable' stops the NTP daemon but does NOT remove server entries, authentication keys, trusted-key designations, source-interface, or VRF binding. Re-issuing 'ntp enable' resumes synchronisation immediately using the preserved configuration.

Part 2 — NTP Server Configuration

## Overview

An NTP server entry tells the device which time source to contact. Each entry specifies the server address (IPv4, IPv6, or FQDN), optional protocol version (3 or 4, default 4), association type (server for unicast or pool for DNS-based pool), iburst for faster initial synchronisation, an authentication key binding, and the prefer flag to prioritise one server over others. Multiple server entries can coexist simultaneously.

## Scenario

Configure DUT1 with a primary NTP server (192.168.100.10) using all available options, add a secondary server, and verify both appear correctly in 'show ntp server'. Then remove one server and confirm clean removal.

## DUT1 — NTP Server Configuration

| Step 1 | Enable NTP and add a basic server (IPv4) |
| --- | --- |

configure terminal ntp enable ! Minimum required: address only (version 4, server mode, no iburst) ntp server 192.168.100.10 exit show ntp server

NTP Servers: Address Version Association Iburst Prefer Key 192.168.100.10 4 server disabled False -

| Step 2 | Add server with all options combined |
| --- | --- |

configure terminal ! version 4, server mode, iburst on, prefer this server ntp server 192.168.100.10 version 4 association server iburst prefer exit show ntp server

| Step 3 | Add a secondary server (version 3) |
| --- | --- |

configure terminal ntp server 192.168.100.11 version 3 exit

| Step 4 | Add an IPv6 NTP server |
| --- | --- |

configure terminal ntp server 2001:db8::1 version 4 iburst exit

| Step 5 | Add a pool-type server using FQDN |
| --- | --- |

configure terminal ! association pool resolves the hostname to multiple pool members ntp server pool.ntp.org association pool iburst exit

| Step 6 | Verify all server entries |
| --- | --- |

show ntp server

NTP Servers: Address Version Association Iburst Prefer Key 192.168.100.10 4 server enabled True - 192.168.100.11 3 server disabled False - 2001:db8::1 4 server enabled False - pool.ntp.org 4 pool enabled False -

| Step 7 | Remove a specific server |
| --- | --- |

configure terminal no ntp server 192.168.100.11 exit show ntp server

NTP Servers: Address Version Association Iburst Prefer Key 192.168.100.10 4 server enabled True - 2001:db8::1 4 server enabled False - pool.ntp.org 4 pool enabled False -

NOTE: The 'iburst' flag causes the device to send a burst of 8 NTP packets at startup, reducing initial synchronisation time from ~60 seconds to ~15 seconds. The 'prefer' flag biases server selection; the preferred server is chosen when its quality metrics are within an acceptable range of the best available source. Only one VRF can be active for NTP; 'ntp server' entries share the globally configured VRF.

Part 3 — NTP Authentication

## Overview

NTP authentication prevents rogue devices from injecting false time into the network. It uses a symmetric-key MAC appended to NTP packets. Three configuration steps are required: (1) define an authentication key with 'ntp authentication-key', (2) mark it as trusted with 'ntp trusted-key', and (3) enable enforcement with 'ntp authenticate'. Each server entry that should use authentication must also reference a key ID with 'ntp server ... key <id>'. Supported algorithms: md5, sha1, sha256, sha384, sha512.

## Authentication Key Lifecycle

Step 1: Define key Step 2: Mark as trusted ntp authentication-key 1 md5 Pass --> ntp trusted-key 1 ¦ v Step 3: Enable enforcement Step 4: Bind to server ntp authenticate <------- ntp server X.X.X.X key 1 ¦ v NTP daemon validates MAC on all received packets Packets with wrong/missing key are REJECTED Figure 2 — NTP authentication configuration pipeline

## Part 3a — Authentication Key Management

| Step 1 | Define authentication keys for all supported algorithms |
| --- | --- |

configure terminal ! MD5 key (key-id range: 1-65535) ntp authentication-key 1 md5 MySecret123 ! SHA1 key ntp authentication-key 2 sha1 Sha1Password ! SHA256 key ntp authentication-key 3 sha256 SecurePass456 ! SHA384 key ntp authentication-key 4 sha384 MediumSecret ! SHA512 key ntp authentication-key 5 sha512 BigSecret789 exit

| Step 2 | Mark keys as trusted |
| --- | --- |

configure terminal ! Only trusted keys are accepted when enforcement is active ntp trusted-key 1 ntp trusted-key 3 exit

| Step 3 | Remove a trusted-key designation (without deleting the key) |
| --- | --- |

configure terminal ! Revokes trust but retains key definition no ntp trusted-key 3 exit

| Step 4 | Delete an authentication key |
| --- | --- |

configure terminal ! Removes key definition entirely no ntp authentication-key 5 exit

NOTE: Deleting an authentication key (no ntp authentication-key) does NOT automatically remove associated trusted-key entries or server key bindings. Remove dependent trusted-key and server key references first to avoid an inconsistent state where a server references a key that no longer exists.

## Part 3b — Full Authentication Workflow (MD5)

This scenario demonstrates the complete end-to-end NTP authentication setup using MD5. NTP-SRV must have key 1 / MD5 / 'MySecret123' configured in /etc/chrony.keys before starting this procedure.

| Step 1 | Define key, trust it, enable enforcement |
| --- | --- |

configure terminal ntp enable ! Step 1: define the shared secret ntp authentication-key 1 md5 MySecret123 ! Step 2: designate key 1 as trusted ntp trusted-key 1 ! Step 3: enable mandatory authentication enforcement ntp authenticate exit

| Step 2 | Bind the key to the server entry |
| --- | --- |

configure terminal ! Step 4: attach key 1 to the server ntp server 192.168.100.10 version 4 iburst key 1 exit show ntp global show ntp server

NTP Servers: Address Version Association Iburst Prefer Key 192.168.100.10 4 server enabled False 1

| Step 3 | Verify show ntp global shows Authentication: True |
| --- | --- |

show ntp global

NTP Configuration: Enabled: True Authentication: True Vrf: default Source Interface: -

| Step 4 | Wait for synchronisation and check associations |
| --- | --- |

! Allow up to 60 seconds for the * (selected) prefix to appear show ntp associations

NTP Associations: refid st t when poll reach delay offset jitter ================================================================= *192.168.100.10 2 u 15 64 377 10.234 -0.233 1.243 * = selected synchronisation source st = stratum (2 = secondary, synced to stratum-1 source) reach = 377 (octal) = all 8 consecutive polls received

| Step 5 | Disable authentication enforcement (keys are preserved) |
| --- | --- |

configure terminal no ntp authenticate exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: -

NOTE: When 'ntp authenticate' is enabled, a server configured WITHOUT a 'key' binding will be rejected for synchronisation even if it is reachable. Its stratum will show as 16 (unreachable) in 'show ntp associations'. A wrong password also causes rejection with no * prefix. Use 'no ntp authenticate' to revert to unauthenticated mode while preserving key definitions.

## Part 3c — SHA256 Authentication Workflow

The same pipeline applies for all SHA variants. NTP-SRV must have the matching key in /etc/chrony.keys (key 3 / SHA256 / 'SecurePass456' in this example).

| Step 1 | Configure SHA256 end-to-end authentication |
| --- | --- |

configure terminal ntp authentication-key 3 sha256 SecurePass456 ntp trusted-key 3 ntp authenticate ntp server 192.168.100.10 iburst key 3 exit

| Step 2 | Verify synchronisation |
| --- | --- |

! Allow up to 60 seconds show ntp associations

*192.168.100.10 2 u 22 64 377 9.812 -0.115 0.987 (SHA256 authenticated session — * confirms selected)

Part 4 — NTP Source Interface

## Overview

By default, the source IP address in outgoing NTP packets is selected by the routing table (the primary IP of the outgoing interface toward each server). The 'ntp source-interface' command overrides this, forcing all NTP packets to use the primary IP of the specified interface as their source address. This is commonly used to ensure NTP traffic uses the management interface or a stable loopback address regardless of routing changes. Supported interface types: Ethernet, Loopback, Management, PortChannel, Vlan.

## DUT1 — Source Interface Configuration

| Step 1 | Set source interface to Management0 |
| --- | --- |

configure terminal ntp source-interface Management 0 exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: Management0

| Step 2 | Set source interface to Loopback0 |
| --- | --- |

! Loopback0 must exist with an IP address configure terminal interface Loopback 0 ip address 1.1.1.1/32 exit ntp source-interface Loopback 0 exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: Loopback0

| Step 3 | Set source interface to Ethernet0 |
| --- | --- |

configure terminal ntp source-interface Ethernet 0 exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: Ethernet0

| Step 4 | Set source interface to a VLAN SVI |
| --- | --- |

configure terminal vlan 10 interface Vlan 10 ip address 192.168.10.1/24 exit ntp source-interface Vlan 10 exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: Vlan10

| Step 5 | Remove source interface (revert to routing-table selection) |
| --- | --- |

configure terminal no ntp source-interface exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: -

NOTE: The configured source interface must exist and have an IP address assigned before 'ntp source-interface' is applied; otherwise the command may be rejected or have no effect. The setting applies globally to all server associations. To verify the source IP used in actual NTP packets, capture traffic on NTP-SRV using Scapy and confirm the IP.src field matches the configured interface's primary address.

Part 5 — NTP VRF Binding

## Overview

In SONiC deployments with a dedicated management VRF, NTP traffic should be bound to the 'mgmt' VRF so that it uses the out-of-band management network rather than the data-plane routing table. The 'ntp vrf' command binds all NTP control traffic to the specified VRF. Only one VRF can be active for NTP at a time. Supported values are 'mgmt' (management VRF) and 'default' (data-plane VRF). Removing the binding with 'no ntp vrf' reverts to the default VRF.

## DUT1 — VRF Binding Configuration

| Step 1 | Bind NTP to the management VRF |
| --- | --- |

! Management VRF must exist before applying this command configure terminal ntp vrf mgmt exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: mgmt Source Interface: -

| Step 2 | Bind NTP to the default VRF |
| --- | --- |

configure terminal ntp vrf default exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: -

| Step 3 | Remove VRF binding (reverts to default) |
| --- | --- |

configure terminal no ntp vrf exit show ntp global

NTP Configuration: Enabled: True Authentication: False Vrf: default Source Interface: -

## DUT1 + DUT2 — NTP Sync via Management VRF

Both DUT1 and DUT2 reach NTP-SRV (192.168.100.10) via the management network. Binding NTP to 'mgmt' VRF ensures NTP traffic is isolated from the data plane.

| Step 1 | Configure both DUTs with mgmt VRF and verify sync |
| --- | --- |

! On DUT1 configure terminal ntp enable ntp vrf mgmt ntp server 192.168.100.10 iburst exit ! On DUT2 configure terminal ntp enable ntp vrf mgmt ntp server 192.168.100.10 iburst exit ! Wait 60 seconds then verify on both DUTs show ntp associations

NTP Associations: refid st t when poll reach delay offset jitter ================================================================= *192.168.100.10 2 u 44 64 377 0.892 0.041 0.318

NOTE: If the management VRF is not yet created when 'ntp vrf mgmt' is issued, the command will be rejected with an error. Create the VRF first using 'ip vrf mgmt' before binding NTP to it. Only one VRF binding is active at a time; issuing 'ntp vrf default' after 'ntp vrf mgmt' replaces the previous binding.

Part 6 — Verification & Show Commands

## Overview

Three show commands are available for NTP: 'show ntp global' displays the administrative configuration state; 'show ntp server' lists all configured server entries; and 'show ntp associations' shows the live runtime synchronisation state from the NTP daemon, including the currently selected source (marked with *), stratum, reach register, and timing metrics.

## show ntp global — Full parameter example

| Step 1 | Configure all global NTP parameters and verify |
| --- | --- |

configure terminal ntp enable ntp authenticate ntp vrf mgmt ntp source-interface Management 0 exit show ntp global

NTP Configuration: Enabled: True Authentication: True Vrf: mgmt Source Interface: Management0

## show ntp server — Multiple entries with all options

| Step 1 | Configure three servers with varied options |
| --- | --- |

configure terminal ntp authentication-key 1 md5 MySecret123 ntp trusted-key 1 ntp server 192.168.100.10 version 4 association server iburst key 1 prefer ntp server 192.168.100.11 version 3 ntp server 2001:db8::1 association pool iburst exit show ntp server

NTP Servers: Address Version Association Iburst Prefer Key 192.168.100.10 4 server enabled True 1 192.168.100.11 3 server disabled False - 2001:db8::1 4 pool enabled False -

## show ntp associations — Understanding the output fields

| Step 1 | Read the associations table after synchronisation |
| --- | --- |

! After NTP has been running for at least 2 poll intervals show ntp associations

NTP Associations: refid st t when poll reach delay offset jitter ================================================================= *192.168.100.10 2 u 128 1024 377 10.234 -0.233 1.243 +192.168.100.11 2 u 456 1024 177 12.156 2.123 1.987 2001:db8::1 3 u 64 512 377 18.432 -1.234 2.456 Prefix legend: * = currently selected synchronisation source + = acceptable candidate peer (could be chosen) (space) = configured but not currently selected Field legend: refid = reference identifier (IP of upstream source) st = stratum (1=GPS/atomic, 2=synced to st1, 16=unreachable) t = association type (u=unicast) when = seconds since last packet received poll = poll interval in seconds reach = octal reachability register (377 = all 8 polls received) delay = round-trip delay to server in ms offset = time difference between local clock and server in ms jitter = variation in offset measurements in ms

NOTE: A reach value of 377 (octal) means all 8 consecutive polls were answered successfully. A reach of 0 means no response has been received. A stratum of 16 means the server is unreachable or has rejected authentication. Use 'show ntp associations' as the primary tool to diagnose synchronisation issues; 'show ntp global' only shows configured state and does not reflect runtime synchronisation status.

## Scapy Traffic Validation

Run these Scapy scripts on NTP-SRV to capture and validate NTP packets sent by DUT1. Ensure promiscuous mode is enabled: sudo ip link set eth0 promisc on

| Step 1 | Capture NTP packets and verify source IP, version, and mode |
| --- | --- |

# Run on NTP-SRV while DUT1 NTP is active from scapy.all import * from scapy.layers.ntp import NTP pkts = sniff(iface='eth0', filter='udp port 123 and src host 192.168.100.1', count=5, timeout=30) for p in pkts: print(f'Src IP : {p[IP].src}') # must match source-interface IP print(f'Dst Port: {p[UDP].dport}') # must be 123 if p.haslayer(NTP): print(f'NTP ver : {p[NTP].version}') # must match configured version print(f'NTP mode: {p[NTP].mode}') # 3 = client request

Src IP : 192.168.100.1 Dst Port: 123 NTP ver : 4 NTP mode: 3

| Step 2 | Verify NTP traffic stops after 'no ntp enable' |
| --- | --- |

! On DUT1 — disable NTP configure terminal no ntp enable exit # On NTP-SRV — capture for 30 seconds; expect zero packets pkts = sniff(iface='eth0', filter='udp port 123 and src host 192.168.100.1', timeout=30) assert len(pkts) == 0, f'FAIL: {len(pkts)} packets still arriving' print('PASS: No NTP traffic after no ntp enable')

Part 7 — Quick Reference

## NTP Service Commands

| Goal | IS-CLI Command |
| --- | --- |
| Enable NTP service | ntp enable |
| Disable NTP service | no ntp enable |
| Enable authentication enforcement | ntp authenticate |
| Disable authentication enforcement | no ntp authenticate |
| Show global NTP configuration | show ntp global |
| Show configured NTP servers | show ntp server |
| Show runtime NTP associations (sync state) | show ntp associations |
| Show running config NTP section | show running-config ¦ grep ntp |

## NTP Server Commands

| Goal | IS-CLI Command |
| --- | --- |
| Add server (minimal) | ntp server <ip-or-fqdn> |
| Add server with iburst (faster sync) | ntp server <ip> iburst |
| Add server with version 3 | ntp server <ip> version 3 |
| Add server as pool (FQDN) | ntp server <fqdn> association pool |
| Add preferred server | ntp server <ip> prefer |
| Add server with auth key binding | ntp server <ip> key <key-id> |
| Add server with all options | ntp server <ip> version 4 association server iburst key 1 prefer |
| Remove a server | no ntp server <ip-or-fqdn> |

## NTP Authentication Commands

| Goal | IS-CLI Command |
| --- | --- |
| Define MD5 key | ntp authentication-key <id> md5 <password> |
| Define SHA1 key | ntp authentication-key <id> sha1 <password> |
| Define SHA256 key | ntp authentication-key <id> sha256 <password> |
| Define SHA384 key | ntp authentication-key <id> sha384 <password> |
| Define SHA512 key | ntp authentication-key <id> sha512 <password> |
| Mark key as trusted | ntp trusted-key <key-id> |
| Revoke trusted status (keep key) | no ntp trusted-key <key-id> |
| Delete an authentication key | no ntp authentication-key <key-id> |

## NTP Source Interface & VRF Commands

| Goal | IS-CLI Command |
| --- | --- |
| Use Management0 as NTP source | ntp source-interface Management 0 |
| Use Loopback0 as NTP source | ntp source-interface Loopback 0 |
| Use Ethernet0 as NTP source | ntp source-interface Ethernet 0 |
| Use PortChannel1 as NTP source | ntp source-interface PortChannel 1 |
| Use Vlan10 SVI as NTP source | ntp source-interface Vlan 10 |
| Remove source interface (use routing) | no ntp source-interface |
| Bind NTP to management VRF | ntp vrf mgmt |
| Bind NTP to default VRF | ntp vrf default |
| Remove VRF binding | no ntp vrf |

## Configuration Cheat-Sheet

| Scenario | Key Settings | Commands (in order) |
| --- | --- | --- |
| Basic sync (unauthenticated) | enable + server + iburst | ntp enable; ntp server <ip> iburst |
| Authenticated sync (MD5) | key → trusted-key → authenticate → server key | ntp authentication-key 1 md5 <pass>; ntp trusted-key 1; ntp authenticate; ntp server <ip> key 1 |
| Authenticated sync (SHA256) | sha256 variant | ntp authentication-key 1 sha256 <pass>; ntp trusted-key 1; ntp authenticate; ntp server <ip> key 1 |
| OOB management (mgmt VRF) | vrf mgmt + source Mgmt0 | ntp vrf mgmt; ntp source-interface Management 0; ntp server <ip> |
| Stable source IP (Loopback) | source-interface Loopback | ntp source-interface Loopback 0; ntp server <ip> |
| Preferred + fallback servers | prefer flag on primary | ntp server <primary> prefer iburst; ntp server <secondary> iburst |
| Pool-based sync (FQDN) | association pool | ntp server pool.ntp.org association pool iburst |
| Disable without losing config | no ntp enable only | no ntp enable (preserves server/auth config) |