ACL Configuration Guide L2 (MAC) ACL ¦ L3 (IPv4) ACL SONiC IS-CLI ¦ Step-by-Step Lab Guide

| Platform | SONiC (D-Link) — IS-CLI |
| --- | --- |

| ACL Type | L2 MAC ACL / L3 IPv4 ACL |
| --- | --- |

| CLI Mode | config-mac-acl / config-ipv4-acl |
| --- | --- |

| Applies to | Ethernet port, PortChannel, VLAN, Switch global |
| --- | --- |

Part 1 — L2 MAC ACL Configuration

## Overview

A Layer-2 MAC ACL filters Ethernet frames based on source/destination MAC addresses, EtherType, PCP (Priority Code Point), DEI, and VLAN ID — before any IP processing occurs. It is applied to a switch port, PortChannel, or VLAN interface in the ingress or egress direction.

## Topology

+------------------+ +------------------+ ¦ Node 1 ¦ ¦ Node 2 ¦ ¦ (DUT / Switch) ¦ ¦ (Traffic Gen) ¦ ¦ ¦ ¦ ¦ ¦ Ethernet0 ---+----------+--- Ethernet0 ¦ ¦ 192.168.1.10/24 ¦ ¦ 192.168.1.11/24 ¦ +------------------+ +------------------+ MAC ACL bound (ingress) Scapy sender Figure 1 — L2 ACL test topology

## Scenario

Block all frames sourced from a specific rogue MAC address (00:00:00:AA:BB:CC), allow traffic matching EtherType IP with PCP 5 (voice), and permit all other traffic inbound on Ethernet0.

## Node 1 — Configuration (DUT)

| Step 1 | Create the MAC ACL table |
| --- | --- |

! Enter global configuration mode configure terminal ! Create a named MAC access-list mac access-list L2_DEMO_ACL ! You are now in config-mac-acl mode ! (prompt changes to: config-mac-acl#)

| Step 2 | Add ACL rules (ACEs) inside the MAC ACL |
| --- | --- |

! Rule 10 — Deny rogue device by source MAC seq 10 deny host 00:00:00:AA:BB:CC any ! Rule 20 — Permit voice traffic (EtherType=IP, PCP=5) seq 20 permit any any ethertype ip pcp 5 ! Rule 30 — Permit ARP frames seq 30 permit any any ethertype arp ! Rule 40 — Permit frames on VLAN 100 only seq 40 permit any any vlan 100 ! Rule 50 — Permit all remaining traffic (avoids implicit deny) seq 50 permit any any exit

NOTE: The implicit deny-all at the end of every MAC ACL drops all unmatched frames. Always add 'seq N permit any any' as the final rule if you want remaining traffic to pass.

| Step 3 | Verify the MAC ACL rules |
| --- | --- |

show mac access-lists L2_DEMO_ACL

MAC access list L2_DEMO_ACL seq 10 deny host 00:00:00:aa:bb:cc any seq 20 permit any any ethertype ip pcp 5 seq 30 permit any any ethertype arp seq 40 permit any any vlan 100 seq 50 permit any any

| Step 4 | Bind the MAC ACL to an interface (inbound) |
| --- | --- |

! Enter the interface context interface Ethernet0 ! Apply MAC ACL in ingress direction mac access-group L2_DEMO_ACL in exit

NOTE: Use 'mac access-group <name> out' to filter egress frames. A single ACL can be applied to multiple interfaces; the same ACL name is reused.

| Step 5 | Verify ACL binding on the interface |
| --- | --- |

show mac access-lists L2_DEMO_ACL interface Ethernet 0

MAC access list L2_DEMO_ACL Bound to: Ethernet0 (ingress) seq 10 deny host 00:00:00:aa:bb:cc any seq 20 permit any any ethertype ip pcp 5 seq 30 permit any any ethertype arp seq 40 permit any any vlan 100 seq 50 permit any any

| Step 6 | Verify ACL groups on interfaces |
| --- | --- |

show mac access-group

Interface Direction MAC ACL Name ----------- --------- --------------- Ethernet0 ingress L2_DEMO_ACL

| Step 7 | Check running configuration |
| --- | --- |

show running-configuration mac access-list L2_DEMO_ACL

mac access-list L2_DEMO_ACL seq 10 deny host 00:00:00:aa:bb:cc any seq 20 permit any any ethertype ip pcp 5 seq 30 permit any any ethertype arp seq 40 permit any any vlan 100 seq 50 permit any any

## Node 2 — Traffic Generation

| Step 1 | Create a Scapy Python script to send test frames |
| --- | --- |

# vim scapy_l2.py from scapy.all import * # Test 1 — rogue MAC (should be DROPPED by seq 10) rogue = Ether(src='00:00:00:aa:bb:cc', dst='ff:ff:ff:ff:ff:ff') sendp(rogue, iface='Ethernet0', count=50) # Test 2 — valid voice frame (should be PERMITTED by seq 20) valid = Ether(src='5c:07:58:6d:7f:c0', dst='c8:78:7d:9b:67:f6') valid = valid / Dot1Q(prio=5, vlan=1) / IP(src='192.168.1.11', dst='192.168.1.10') sendp(valid, iface='Ethernet0', count=50)

| Step 2 | Configure IP address on the test interface |
| --- | --- |

sudo config interface ip add Ethernet0 192.168.1.11/24

## Result — Verify Counters on Node 1

After running the Scapy script on Node 2, check the interface counters on Node 1 to confirm the rogue frames were dropped and the valid frames were forwarded:

show interfaces counters -i Ethernet0

IFACE STATE RX_OK RX_BPS RX_ERR RX_DRP TX_OK TX_BPS ---------- ----- ----- -------- ------ ------ ----- -------- Ethernet0 U 100 1.20 KB/s 0 50 50 0.60 KB/s RX_OK=100 (50 rogue + 50 valid received at NIC) RX_DRP=50 (rogue frames silently dropped by ACL seq 10) TX_OK=50 (valid voice frames forwarded through)

NOTE: RX_DRP counter increments for frames dropped by an ingress ACL. If RX_DRP does not increase, verify the ACL is correctly bound with 'show mac access-group'.

Part 2 — L3 IPv4 ACL Configuration

## Overview

A Layer-3 IP ACL filters packets based on source/destination IP addresses, IP protocol (TCP, UDP, ICMP), and port numbers. It operates on routed traffic and is applied to a routed interface (Ethernet, PortChannel, VLAN SVI) or globally to the Switch.

## Topology

+------------------+ +------------------+ ¦ Node 1 ¦ ¦ Node 2 ¦ ¦ (DUT / Router) ¦ ¦ (Traffic Gen) ¦ ¦ ¦ ¦ ¦ ¦ Ethernet27 ---+----------+--- Ethernet52 ¦ ¦ 192.168.1.10/24 ¦ ¦ 192.168.1.11/24 ¦ +------------------+ +------------------+ IP ACL bound (ingress) Scapy sender Figure 2 — L3 ACL test topology

## Scenario

Drop all TCP traffic from 10.0.0.2/32, permit HTTP and HTTPS TCP, block Telnet (port 23), permit UDP DNS (port 53), allow ICMP, and permit all remaining IP traffic inbound on Ethernet27.

## Node 1 — Configuration (DUT)

| Step 1 | Create the IP ACL table |
| --- | --- |

! Enter global configuration mode configure terminal ! Create a named IPv4 access-list ip access-list L3_DEMO_ACL ! You are now in config-ipv4-acl mode

| Step 2 | Add ACL rules (ACEs) inside the IP ACL |
| --- | --- |

! Rule 10 — Drop all traffic from rogue host 10.0.0.2 seq 10 deny ip host 10.0.0.2 any ! Rule 20 — Block Telnet (TCP port 23) from any source seq 20 deny tcp any any eq 23 ! Rule 30 — Permit HTTP (TCP 80) and HTTPS (TCP 443) seq 30 permit tcp any any range 80 443 ! Rule 40 — Permit UDP DNS (port 53) to specific server seq 40 permit udp any host 192.168.1.10 eq 53 ! Rule 50 — Permit ICMP (ping) from any source seq 50 permit icmp any any ! Rule 60 — Permit remaining IP traffic seq 60 permit ip any any exit

NOTE: Rules are matched top-to-bottom. Place more-specific rules (host matches) BEFORE general rules (subnet or any). The implicit deny-all applies after the last rule.

| Step 3 | Configure IP address on the ingress interface |
| --- | --- |

sudo config interface ip add Ethernet27 192.168.1.10/24

| Step 4 | Verify the IP ACL rules |
| --- | --- |

show ip access-lists L3_DEMO_ACL

IP access list L3_DEMO_ACL seq 10 deny ip host 10.0.0.2 any seq 20 deny tcp any any eq 23 seq 30 permit tcp any any range 80 443 seq 40 permit udp any host 192.168.1.10 eq 53 seq 50 permit icmp any any seq 60 permit ip any any

| Step 5 | Bind the IP ACL to the interface (inbound) |
| --- | --- |

interface Ethernet27 ! Apply IP ACL in ingress direction ip access-group L3_DEMO_ACL in exit

NOTE: To filter outbound routed traffic, use 'ip access-group <name> out'. IPv6 uses 'ipv6 access-group' with an ipv6 access-list.

| Step 6 | Verify binding on the interface |
| --- | --- |

show ip access-lists L3_DEMO_ACL interface Ethernet 27

IP access list L3_DEMO_ACL Bound to: Ethernet27 (ingress) seq 10 deny ip host 10.0.0.2 any seq 20 deny tcp any any eq 23 seq 30 permit tcp any any range 80 443 seq 40 permit udp any host 192.168.1.10 eq 53 seq 50 permit icmp any any seq 60 permit ip any any

| Step 7 | Verify all IP ACL bindings |
| --- | --- |

show ip access-group

Interface Direction IP ACL Name ----------- --------- --------------- Ethernet27 ingress L3_DEMO_ACL

| Step 8 | Check running configuration |
| --- | --- |

show running-configuration ip access-list L3_DEMO_ACL

{ "L3_DEMO_ACL¦RULE_1": { "IP_PROTOCOL": "IP", "PACKET_ACTION": "DROP", "PRIORITY": "9999", "SRC_IP": "10.0.0.2/32" }, "L3_DEMO_ACL¦RULE_2": { "IP_PROTOCOL": "TCP", "PACKET_ACTION": "DROP", "L4_DST_PORT": "23", "PRIORITY": "9998" }, "L3_DEMO_ACL¦RULE_3": { "IP_PROTOCOL": "TCP", "PACKET_ACTION": "FORWARD", "L4_DST_PORT_RANGE": "80-443", "PRIORITY": "9997" } }

## Node 2 — Traffic Generation

| Step 1 | Create a Scapy Python script to send test packets |
| --- | --- |

# vim scapy_l3.py from scapy.all import * # Test 1 — rogue host (should be DROPPED by seq 10) rogue = Ether() / IP(src='10.0.0.2', dst='192.168.1.10') / TCP(dport=80) sendp(rogue, iface='Ethernet52', count=50) # Test 2 — Telnet traffic (should be DROPPED by seq 20) telnet = Ether() / IP(src='10.0.0.5', dst='192.168.1.10') / TCP(dport=23) sendp(telnet, iface='Ethernet52', count=50) # Test 3 — HTTPS traffic (should be PERMITTED by seq 30) https = Ether() / IP(src='10.0.0.5', dst='192.168.1.10') / TCP(dport=443) sendp(https, iface='Ethernet52', count=50) # Test 4 — ICMP ping (should be PERMITTED by seq 50) ping = Ether() / IP(src='10.0.0.5', dst='192.168.1.10') / ICMP() sendp(ping, iface='Ethernet52', count=50)

| Step 2 | Configure IP address on the test interface |
| --- | --- |

sudo config interface ip add Ethernet52 192.168.1.11/24

## Result — Verify Counters on Node 1

After running the Scapy script, inspect interface counters on Node 1's Ethernet27 to verify drop and forward behaviour:

show interfaces counters -i Ethernet27

IFACE STATE RX_OK RX_BPS RX_ERR RX_DRP TX_OK TX_BPS ---------- ----- ----- --------- ------ ------ ----- --------- Ethernet27 U 200 2.40 KB/s 0 100 100 1.20 KB/s RX_OK=200 (all 4 x 50 frames received at NIC level) RX_DRP=100 (50 rogue + 50 Telnet frames dropped by ACL) TX_OK=100 (HTTPS + ICMP frames forwarded)

NOTE: RX_DRP will increment for both 'deny ip' (seq 10) and 'deny tcp ... eq 23' (seq 20) rules. Use 'show ip access-lists' on supported platforms to see per-rule hit counters.

Part 3 — Quick Reference

## L2 MAC ACL — common rule patterns

| Goal | IS-CLI Command |
| --- | --- |

| Permit all frames | seq N permit any any |
| --- | --- |

| Deny specific source MAC | seq N deny host 00:00:00:AA:BB:CC any |
| --- | --- |

| Permit specific src → dst MAC | seq N permit 00:11:22:33:44:55 any |
| --- | --- |

| Permit only IPv4 EtherType | seq N permit any any ethertype ip |
| --- | --- |

| Permit ARP frames | seq N permit any any ethertype arp |
| --- | --- |

| Permit voice (PCP=5) | seq N permit any any pcp vo |
| --- | --- |

| Permit network-ctrl (PCP=7) | seq N permit any any pcp nc |
| --- | --- |

| Permit PCP value 0 (best-effort) | seq N permit any any pcp be |
| --- | --- |

| Permit DEI=0 only (non-drop) | seq N permit any any dei 0 |
| --- | --- |

| Filter by VLAN tag | seq N permit any any vlan 100 |
| --- | --- |

| Match multi-tagged (QinQ) | seq N permit any any vlan-tag-format multi-tagged |
| --- | --- |

| Delete a specific rule | no seq 10 |
| --- | --- |

## L3 IPv4 ACL — common rule patterns

| Goal | IS-CLI Command |
| --- | --- |

| Permit all IP | seq N permit ip any any |
| --- | --- |

| Deny all IP from a host | seq N deny ip host 10.0.0.2 any |
| --- | --- |

| Deny a subnet | seq N deny ip 10.1.1.0/24 any |
| --- | --- |

| Permit TCP from host to host | seq N permit tcp host 10.1.1.10 host 10.2.2.20 |
| --- | --- |

| Permit HTTP (port 80) | seq N permit tcp any any eq 80 |
| --- | --- |

| Permit HTTP + HTTPS range | seq N permit tcp any any range 80 443 |
| --- | --- |

| Block Telnet (port 23) | seq N deny tcp any any eq 23 |
| --- | --- |

| Permit UDP DNS (port 53) | seq N permit udp any any eq 53 |
| --- | --- |

| Permit ICMP (ping) | seq N permit icmp any any |
| --- | --- |

| Permit specific protocol num | seq N permit 89 any any (89=OSPF) |
| --- | --- |

| Delete a specific rule | no seq 10 |
| --- | --- |

## Binding cheat-sheet

| Target | Enter Context | Bind Command |
| --- | --- | --- |

| Ethernet port | interface Ethernet0 | mac access-group <name> in |
| --- | --- | --- |

| PortChannel / LAG | interface PortChannel1 | mac access-group <name> in |
| --- | --- | --- |

| VLAN SVI | interface Vlan100 | ip access-group <name> in |
| --- | --- | --- |

| Global (Switch) | (global config) | mac access-group <name> in |
| --- | --- | --- |